POWER PROFILE

Complete Technical Documentation

Graduation Project — Electrical Load Analysis System

Author: Ahmed Zoher

Generated: June 7, 2026

Stack: Laravel 13 · React 18 · SQLite · IEC 60364-8-1

# POWER PROFILE — COMPLETE PROJECT DOCUMENTATION

Generated: June 7, 2026

Codebase inventory: 80 PHP files · 44 JS/JSX files · 55 migrations · 9 services · 28 controllers · 17 models · 21 pages · 15 components

## 1. PROJECT OVERVIEW

### What It Is

Power Profile is a full-stack web application for electrical load analysis of buildings and facilities.

An engineer logs in, creates a project, enters every electrical load room-by-room (computers, motors, AC units, lighting), and the system automatically computes:

Total apparent power (kVA), active power (kW), and reactive power (kVAR) with IEC diversity factors

Whether power factor correction is needed and exactly what capacitor bank to install

How solar panels, batteries, a utility grid connection, and a diesel generator interact hour-by-hour

Which hours of the day are cheapest to run shiftable loads (water heaters, HVAC, EV chargers)

A 25-year financial projection showing payback period and net benefit

Phase imbalance and the optimal assignment of single-phase loads across three phases

### Context

Graduation project by Ahmed Zoher, June 2026

Designed for use in the Palestinian territories (Gaza / West Bank) under PENRA regulations

Has startup potential as a SaaS tool for electrical engineering consultancies

### Technology Stack (exact versions)

Backend:

PHP ^8.3

Laravel ^13.0 (framework)

Laravel Sanctum ^4.3 (API token authentication)

Laravel Socialite ^5.26 (Google OAuth 2.0)

Laravel Tinker ^3.0 (REPL)

SQLite (file-based database — no separate DB server required)

Frontend:

React ^18.2.0

React DOM ^18.2.0

React Router DOM ^6.30.3 (client-side routing)

Recharts ^3.8.1 (charts — hourly load profile, 25-year projection)

Axios ^1.13.6 (HTTP client)

xlsx ^0.18.5 / SheetJS (Excel export)

Build / Tooling:

Vite ^4.4.5 (bundler)

Tailwind CSS ^3.4.19 (utility-first CSS)

ESLint ^8.45.0 (linting)

PostCSS ^8.5.8 / Autoprefixer ^10.4.27

### The Four Computation Engines

User enters loads

↓

┌───────────────────┐

│  1. POWER ENGINE  │  TotalPowerController

│  Diversity + PF   │  DiversityFactorService

│  Motor inrush     │  SocketDemandService

└────────┬──────────┘

↓

┌───────────────────┐

│ 2. DISPATCH ENGINE│  SourceDispatchService

│  Solar/Batt/Grid  │  SolarIrradianceService

│  Hour-by-hour     │  CostSignalService

└────────┬──────────┘

↓

┌───────────────────┐

│ 3. OPTIMIZE ENGINE│  ProjectController::optimizeShiftable()

│  Shift loads to   │  CostSignalService (signal builder)

│  cheapest hours   │  ScheduleController (display)

└────────┬──────────┘

↓

┌───────────────────┐

│ 4. FINANCIAL ENG. │  FinancialAnalysisService

│  Savings, LCOE,   │  FinancialController

│  25-yr projection │

└───────────────────┘

### Electrical Standards Implemented

| Standard | Where Applied |
| --- | --- |
| IEC 60364-8-1 | Diversity factors for buildings/rooms; default PF correction threshold 0.85 |
| PENRA (Palestinian Energy Authority) | Min PF = 0.85, target PF = 0.95 |
| NEC Article 430 | Motor inrush 125% rule for largest motor |
| IEC 60831 | Capacitor bank sizing; delta (Δ) configuration |
| ISO 8528 | Affine diesel generator fuel consumption model F(P) = F₀ + slope×P |
| CIBSE Guide C | Per-room coincidence factors; building-type diversity factors |
| NASA POWER | Satellite GHI data for solar irradiance (free, global, ±3% accuracy) |

## 2. COMPLETE FILE INVENTORY

### Services (backend/app/Services/)

| File | Purpose |
| --- | --- |
| DiversityFactorService.php | IEC 60364-8-1 diversity factor lookup tables (per building type, per room type) |
| SocketDemandService.php | 3-tier demand factor for outlet sockets; building coincidence factor |
| SolarIrradianceService.php | Hourly solar output via NASA POWER API primary + sinusoidal PSH-table fallback |
| BatteryChemistryService.php | Preset parameter tables for 5 battery chemistries (LFP, NMC, lead-acid variants) |
| SourceDispatchService.php | 7-step priority dispatch (solar → batteries → grid → generator) for all 24 hours |
| CostSignalService.php | 24-element marginal cost signal ($/kWh per hour) from tariff + battery + solar surplus |
| FinancialAnalysisService.php | Dual-dispatch baseline + solar comparison, annual costs, savings, LCOE, 25-yr projection |
| BackupExporter.php | JSON export of complete project data for portable backup/restore |
| ValidationReferenceService.php | Static reference data (standards tables, NEC/IEC limits) used by the validation page |

### Controllers (backend/app/Http/Controllers/Api/)

| File | Purpose |
| --- | --- |
| AdminController.php | Admin dashboard data: user list, project list, system stats |
| BatteryController.php | CRUD for battery banks; chemistry preset loader |
| BuildingComponentController.php | CRUD for components attached at building level |
| BuildingController.php | CRUD for buildings within a project |
| ComponentTypeController.php | CRUD for reusable component type catalog |
| CostSignalController.php | Computes and returns 24-hour cost signal for a project |
| FinancialController.php | Runs FinancialAnalysisService and returns structured JSON |
| FloorComponentController.php | CRUD for components at floor level |
| FloorController.php | CRUD for floors within a building |
| GeneratorLineController.php | CRUD for generator power sources |
| LoadProfileController.php | Returns hourly load profile (24-element W array) with diversity |
| NavigationController.php | Breadcrumb path data for a given entity |
| PhaseBalanceController.php | Phase balance analysis + greedy optimal assignment at building/floor |
| ProjectBackupController.php | Client-side JSON project export and import |
| ProjectComponentController.php | CRUD for components at project level |
| ProjectController.php | Project CRUD; shiftable-load optimizer; defense summary |
| ProjectMemberController.php | Add/remove/update roles for project collaborators |
| RoomComponentController.php | CRUD for components at room level |
| RoomController.php | CRUD for rooms within a floor |
| ScheduleController.php | 24-hour dispatch result with source breakdown |
| ServerBackupController.php | Server-side backup create/download/delete |
| SocketController.php | CRUD for outlet socket records (polymorphic) |
| SolarSystemController.php | CRUD for named solar PV systems |
| TotalPowerController.php | Full power analysis: diversity, PF correction, inrush, battery summary |
| UserController.php | Current user profile; update name |
| UtilityLineController.php | CRUD for utility grid connections with tariff data |
| ValidationController.php | Returns IEC/NEC reference standards for the validation page |
| GoogleController.php | Google OAuth 2.0 redirect and callback |

### Auth & Middleware

| File | Purpose |
| --- | --- |
| Controller.php | Base controller (empty — Laravel default) |
| AdminMiddleware.php | Restricts routes to users with is_admin = true |
| SecurityHeaders.php | Adds HTTP security headers to every response |
| Auth/GoogleController.php | Handles Google OAuth redirect and callback |

### Models (backend/app/Models/)

| File | Table | Purpose |
| --- | --- | --- |
| User.php | users | Authenticated users (Google OAuth + admin) |
| Project.php | projects | Top-level project container |
| Building.php | buildings | Building within a project |
| Floor.php | floors | Floor within a building |
| Room.php | rooms | Room within a floor |
| ComponentType.php | component_types | Reusable catalog of electrical component types |
| ProjectComponent.php | project_components | Component instance at project level |
| BuildingComponent.php | building_components | Component instance at building level |
| FloorComponent.php | floor_components | Component instance at floor level |
| RoomComponent.php | room_components | Component instance at room level |
| UtilityLine.php | utility_lines | Grid utility connection (polymorphic) |
| GeneratorLine.php | generator_lines | Diesel generator (polymorphic) |
| Socket.php | sockets | Outlet socket records (polymorphic) |
| Battery.php | batteries | Battery bank linked to a project |
| SolarSystem.php | solar_systems | Named solar PV system linked to a project |
| ProjectUser.php | project_users | Many-to-many: project ↔ user with role |
| ServerBackup.php | server_backups | Metadata for server-side backup files |

### Form Requests (backend/app/Http/Requests/)

| File | Validates |
| --- | --- |
| StoreProjectRequest.php | Create project |
| UpdateProjectRequest.php | Update project |
| StoreBuildingRequest.php | Create building |
| UpdateBuildingRequest.php | Update building |
| StoreFloorRequest.php | Create floor |
| UpdateFloorRequest.php | Update floor |
| StoreRoomRequest.php | Create room |
| UpdateRoomRequest.php | Update room |
| StoreComponentRequest.php | Create component (all levels) |
| UpdateComponentRequest.php | Update component |
| StoreUtilityLineRequest.php | Create utility line |
| UpdateUtilityLineRequest.php | Update utility line |
| StoreGeneratorLineRequest.php | Create generator line |
| UpdateGeneratorLineRequest.php | Update generator line |
| StoreBatteryRequest.php | Create battery |
| UpdateBatteryRequest.php | Update battery |
| StoreSocketRequest.php | Create socket record |
| UpdateSocketRequest.php | Update socket record |
| ApiRequest.php | Base form request (shared auth) |

### Frontend Pages (frontend/src/pages/)

| File | Route | Purpose |
| --- | --- | --- |
| LoginPage.jsx | /login | Google OAuth login button |
| AuthCallbackPage.jsx | /auth/callback | Handles token from OAuth callback URL |
| DashboardPage.jsx | /dashboard | Project list with total kW/kVA per project |
| CreateProjectPage.jsx | /projects/new | Create new project form |
| ProjectPage.jsx | /projects/:id | Project overview: PowerBanner + sources |
| BuildingPage.jsx | /projects/:id/buildings/:bid | Building detail with components and power |
| BuildingsPage.jsx | (sub-page) | Building list within project |
| FloorPage.jsx | /projects/:id/buildings/:bid/floors/:fid | Floor detail |
| FloorsPage.jsx | (sub-page) | Floor list within building |
| NewRoomPage.jsx | /projects/:id/buildings/:bid/floors/:fid/rooms/:rid | Room detail with components |
| RoomPage.jsx | (sub-page) | Room list within floor |
| RoomDetailPage.jsx | (sub-page) | Alternate room detail view |
| LoadSchedulePage.jsx | /projects/:id/schedule | Load schedule: base/optimized/combined tabs |
| PhaseBalancePage.jsx | /projects/:id/phase-balance | Phase balance analysis |
| FinancialPage.jsx | /projects/:id/financial | Financial analysis: costs, savings, 25yr chart |
| ValidationPage.jsx | /validation | IEC/NEC reference standards browser |
| DefensePrepPage.jsx | /defense-prep | Admin-only exam Q&A accordion (42 questions) |
| SingleLineDiagramPage.jsx | /projects/:id/single-line | Auto-generated SVG single-line diagram |
| AdminLoginPage.jsx | /admin/login | Admin-only login with email/password |
| AdminDashboardPage.jsx | /admin/dashboard | Admin panel: users, projects, backups |

### Frontend Components (frontend/src/components/)

| File | Purpose |
| --- | --- |
| Navbar.jsx | Top navigation bar with user menu and Defense Prep button (admin only) |
| PowerBanner.jsx | MAX LOAD / OPTIMIZED kVA/kW display with PDF and Excel export buttons |
| PowerSourcesBanner.jsx | Summary strip showing solar capacity, battery, grid, generator |
| ProjectSidebar.jsx | Left nav for all project sub-pages |
| EntityComponents.jsx | Reusable component CRUD table used on all entity pages |
| EntityScheduleModal.jsx | Modal for editing entity-level schedule (work days, season intervals) |
| EntitySockets.jsx | Socket outlet management for any entity |
| TimeScheduleModal.jsx | Modal for editing component time intervals (usage_time_intervals) |
| ReactivePowerPanel.jsx | Capacitor bank sizing panel shown when PF < 0.85 |
| BackupChoiceModal.jsx | Modal: choose between client (JSON) or server backup |
| ProjectMembersModal.jsx | Manage project member roles |
| ServerBackupsList.jsx | List of server-side backups with download/delete |
| UserCard.jsx | User avatar + name display |
| ErrorBoundary.jsx | Catches React render errors; shows fallback UI |
| LoadingSpinner.jsx | Centered spinner shown during lazy-loaded page suspense |

### Utilities & Context (frontend/src/)

| File | Purpose |
| --- | --- |
| contexts/AuthContext.jsx | React context: user state, login, logout, isLoading |
| layouts/ProjectLayout.jsx | Wrapper for all /projects/:id routes; fetches project data |
| api/axios.js | Configured Axios instance with base URL and auth token header |
| utils/exportToExcel.js | 3-sheet SheetJS export (Summary, Components, Financial) |
| utils/printPowerReport.js | print-to-PDF HTML template with letterhead and power summary |
| utils/downloadJson.js | Triggers browser file download for JSON backup |
| utils/navContext.js | Navigation context utilities |
| main.jsx | React entry point (ReactDOM.createRoot) |
| App.jsx | All routes, ProtectedRoute/PublicRoute guards, lazy imports |

Source: All files listed above.

## 3. DATABASE SCHEMA — COMPLETE

### Table: users

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint unsigned | NO | auto | Primary key |
| name | varchar(255) | NO |  | Display name |
| email | varchar(255) | NO |  | Unique, used for login |
| email_verified_at | timestamp | YES | NULL | OAuth users are auto-verified |
| password | varchar(255) | YES | NULL | NULL for OAuth-only users |
| remember_token | varchar(100) | YES | NULL | Laravel session token |
| google_id | text | YES | NULL | Google sub ID (widened from varchar) |
| google_token | text | YES | NULL | OAuth access token (widened) |
| google_refresh_token | text | YES | NULL | OAuth refresh token (widened) |
| avatar | varchar(255) | YES | NULL | Google profile picture URL |
| is_admin | boolean | NO | false | Grants admin dashboard access |
| created_at / updated_at | timestamp | YES | NULL | Standard timestamps |

Notes: google_id, google_token, google_refresh_token were widened from varchar(255) to text in migration 2026_05_17 to accommodate long Google token strings.

### Table: projects

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint unsigned | NO | auto | Primary key |
| user_id | bigint unsigned | NO |  | FK → users.id (owner) |
| name | varchar(255) | NO |  | Project name |
| description | text | YES | NULL | Free-text description |
| area | decimal(10,2) | YES | NULL | Total site area m² |
| voltage | varchar(50) | YES | NULL | System voltage (e.g. '400/230V') |
| frequency | integer | YES | NULL | Hz (50 or 60) |
| currency_symbol | varchar(10) | NO | '$' | Display currency symbol |
| solar_source | enum('max','existing') | NO | 'max' | How solar capacity is determined |
| existing_solar_power | decimal(12,2) | YES | NULL | VA if solar_source='existing' |
| generator_source | boolean | NO | false | Whether generator is a source |
| location_lat | decimal(10,7) | YES | NULL | Latitude for NASA POWER API |
| location_lng | decimal(10,7) | YES | NULL | Longitude for NASA POWER API |
| work_days | json | YES | NULL | Array of day names, e.g. ["monday","tuesday",...] |
| working_season_intervals | json | YES | NULL | Array of {from,to} date-range objects |
| auto_backup | boolean | NO | false | Enable daily auto-backup |
| created_at / updated_at | timestamp |  |  |  |

### Table: buildings

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| project_id | bigint | NO |  | FK → projects.id |
| name | varchar(255) | NO |  | Building name |
| area | decimal(10,2) | YES | NULL | Floor area m² (used for solar capacity estimate) |
| type | varchar(100) | YES | NULL | e.g. 'office', 'hospital', 'residential_apartment' — drives diversity factors |
| existing_solar_power | decimal(12,2) | YES | NULL | Existing solar VA on this building |
| work_days | json | YES | NULL | Overrides project-level work days |
| working_season_intervals | json | YES | NULL | Overrides project-level season |
| created_at / updated_at | timestamp |  |  |  |

### Table: floors

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| building_id | bigint | NO |  | FK → buildings.id |
| name | varchar(255) | NO |  | Floor name/number |
| work_days | json | YES | NULL | Overrides building-level work days |
| working_season_intervals | json | YES | NULL |  |
| created_at / updated_at | timestamp |  |  |  |

### Table: rooms

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| floor_id | bigint | NO |  | FK → floors.id |
| name | varchar(255) | NO |  | Room name |
| type | varchar(100) | YES | NULL | e.g. 'bedroom', 'server_room' — drives room coincidence factor |
| work_days | json | YES | NULL |  |
| working_season_intervals | json | YES | NULL |  |
| created_at / updated_at | timestamp |  |  |  |

### Table: component_types

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| name | varchar(255) | NO |  | e.g. 'LED Lamp', 'Air Conditioner' |
| default_power | decimal(12,2) | YES | NULL | Suggested VA for this type |
| default_power_factor | decimal(4,3) | YES | NULL | Suggested PF (0.001–1.000) |
| is_motor | boolean | NO | false | Marks motor-type loads for inrush rule |
| created_at / updated_at | timestamp |  |  |  |

### Component Tables (4 tables, same structure)

Tables: project_components, building_components, floor_components, room_components

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| {entity}_id | bigint | NO |  | FK to parent entity (project_id / building_id / floor_id / room_id) |
| component_type_id | bigint | NO |  | FK → component_types.id |
| component_name | varchar(255) | YES | NULL | Optional override name |
| power | decimal(12,2) | NO |  | Apparent power in VA (nameplate rating) |
| power_factor | decimal(4,3) | NO | 1.000 | PF (0.01–1.0) |
| quantity | integer | NO | 1 | Number of identical units |
| phases | enum('1phase','3phase') | NO | '1phase' | Single or three phase |
| phase | char(1) | YES | NULL | Assigned phase: 'A', 'B', or 'C' |
| priority | enum('critical','essential','normal') | NO | 'normal' | Load priority class |
| group_name | varchar(255) | YES | NULL | N+1 redundancy group identifier |
| is_motor | boolean | NO | false | True = NEC inrush rule applies |
| usage_season | enum('all','summer','winter','spring','autumn') | NO | 'all' | Season filter |
| usage_day_type | enum('all','weekday','weekend') | NO | 'all' | Day filter |
| usage_time_intervals | json | YES | NULL | Array of {start,end} time windows |
| load_flexibility | enum('fixed','shiftable','curtailable') | NO | 'fixed' | For optimizer |
| required_run_hours | integer | YES | NULL | Hours/day the load must run |
| earliest_start_hour | integer | YES | NULL | Earliest allowed start (0–23) |
| latest_end_hour | integer | YES | NULL | Latest must-finish hour (1–24) |
| max_interruptions | integer | YES | NULL | Max separate time windows (0=continuous) |
| created_at / updated_at | timestamp |  |  |  |

Note on `power` column: Stores VA (apparent power), NOT watts. Active power W = power × power_factor. This is intentional — equipment nameplate ratings are in kVA.

### Table: utility_lines

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| utilizable_type | varchar(255) | NO |  | Polymorphic type ('App\Models\Project' etc.) |
| utilizable_id | bigint | NO |  | Polymorphic FK |
| name | varchar(255) | YES | NULL | Label e.g. 'Main Grid' |
| power | decimal(12,2) | YES | NULL | Rated capacity in VA |
| phases | enum('1phase','3phase') | NO | '3phase' |  |
| tariff_per_kwh | decimal(10,4) | YES | NULL | Base electricity rate $/kWh |
| peak_tariff_per_kwh | decimal(10,4) | YES | NULL | Peak-hours rate (NULL = flat rate) |
| peak_hours_start | integer | YES | NULL | Hour 0–23 when peak period starts |
| peak_hours_end | integer | YES | NULL | Hour 1–24 when peak period ends |
| created_at / updated_at | timestamp |  |  |  |

### Table: generator_lines

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| generable_type | varchar(255) | NO |  | Polymorphic type |
| generable_id | bigint | NO |  | Polymorphic FK |
| name | varchar(255) | YES | NULL |  |
| power | decimal(12,2) | YES | NULL | Rated output VA |
| phases | varchar(10) | YES | NULL | '1phase' or '3phase' |
| fuel_cost_per_liter | decimal(10,4) | YES | NULL | Fuel price $/L |
| fuel_consumption_lph | decimal(10,4) | YES | NULL | L/hr at 100% rated load |
| no_load_fuel_lph | decimal(10,4) | YES | NULL | L/hr at 0% load (NULL → 30% of rated) |
| min_load_pct | integer | YES | NULL | Minimum recommended load % |
| optimal_load_pct | integer | YES | NULL | Most efficient load % |
| created_at / updated_at | timestamp |  |  |  |

### Table: sockets

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| socketable_type | varchar(255) | NO |  | Polymorphic: Project/Building/Floor/Room |
| socketable_id | bigint | NO |  | Polymorphic FK |
| quantity | integer | NO | 1 | Number of outlet points |
| created_at / updated_at | timestamp |  |  |  |

### Table: batteries

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| project_id | bigint | NO |  | FK → projects.id |
| solar_system_id | bigint | YES | NULL | If paired to a solar system |
| name | varchar(255) | NO |  | Label e.g. 'Main BESS' |
| chemistry | varchar(50) | NO |  | 'lithium_lfp', 'lead_acid_flooded', etc. |
| nominal_capacity_kwh | decimal(10,2) | NO |  | Nameplate capacity kWh |
| depth_of_discharge | decimal(4,3) | NO |  | DoD (0.0–1.0) |
| usable_capacity_kwh | decimal(10,2) | NO |  | = nominal × DoD (stored computed) |
| round_trip_efficiency | decimal(4,3) | NO |  | RTE (0.0–1.0) |
| max_charge_power_kw | decimal(10,2) | NO |  | Max charge rate kW |
| max_discharge_power_kw | decimal(10,2) | NO |  | Max discharge rate kW |
| current_soc | decimal(4,3) | NO | 1.000 | Current state of charge |
| rated_cycle_life | integer | NO |  | Full cycles before 80% capacity |
| age_years | decimal(5,2) | NO | 0 | Current age for replacement scheduling |
| is_active | boolean | NO | true | Include in dispatch simulation |
| purchase_cost | decimal(12,2) | YES | NULL | Initial purchase cost $ |
| replacement_cost | decimal(12,2) | YES | NULL | Replacement cost $ |
| annual_maintenance_cost | decimal(10,2) | YES | NULL | $/year maintenance |
| created_at / updated_at | timestamp |  |  |  |

Computed properties (not stored):

current_available_kwh = usable_capacity_kwh × current_soc

health_status: 'good' / 'degraded' / 'replace' (based on age vs rated cycle life)

### Table: solar_systems

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| project_id | bigint | NO |  | FK → projects.id |
| name | varchar(255) | NO |  | e.g. 'Rooftop South Array' |
| capacity_kw | decimal(10,2) | NO |  | Nameplate DC peak capacity kW |
| is_active | boolean | NO | true | Include in dispatch |
| installation_cost | decimal(12,2) | YES | NULL | $ upfront |
| annual_maintenance_cost | decimal(10,2) | YES | NULL | $/year |
| created_at / updated_at | timestamp |  |  |  |

### Table: project_users

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| project_id | bigint | NO |  | FK → projects.id |
| user_id | bigint | NO |  | FK → users.id |
| role | enum('viewer','main','admin') | NO | 'viewer' | Access level within this project |
| created_at / updated_at | timestamp |  |  |  |

Roles:

viewer — read-only (can see data, cannot edit)

main — can edit everything except delete project / manage members

admin — full control including member management

### Table: server_backups

| Column | Type | Nullable | Default | Notes |
| --- | --- | --- | --- | --- |
| id | bigint | NO | auto | PK |
| project_id | bigint | NO |  | FK → projects.id |
| filename | varchar(255) | NO |  | Stored file name on server |
| size_bytes | bigint | YES | NULL | File size |
| created_at / updated_at | timestamp |  |  |  |

Source: All migration files in backend/database/migrations/

## 4. SERVICES — COMPLETE DOCUMENTATION

### 4.1 DiversityFactorService

File: backend/app/Services/DiversityFactorService.php

Purpose: Provides IEC 60364-8-1 and CIBSE Guide C diversity factor values keyed by building type and room type.

#### Constants

BUILDING_DFS = [

'residential_house'      => ['room_to_floor' => 0.60, 'floor_to_building' => 0.70],

'residential_apartment'  => ['room_to_floor' => 0.65, 'floor_to_building' => 0.70],

'hotel'                  => ['room_to_floor' => 0.65, 'floor_to_building' => 0.70],

'office'                 => ['room_to_floor' => 0.85, 'floor_to_building' => 0.80],

'educational_school'     => ['room_to_floor' => 0.80, 'floor_to_building' => 0.80],

'educational_university' => ['room_to_floor' => 0.85, 'floor_to_building' => 0.80],

'retail'                 => ['room_to_floor' => 0.85, 'floor_to_building' => 0.85],

'hospital'               => ['room_to_floor' => 0.90, 'floor_to_building' => 0.90],

'industrial'             => ['room_to_floor' => 0.85, 'floor_to_building' => 0.85],

'mosque_worship'         => ['room_to_floor' => 0.80, 'floor_to_building' => 0.75],

'sports'                 => ['room_to_floor' => 0.80, 'floor_to_building' => 0.80],

]

DEFAULT_BUILDING_DFS = ['room_to_floor' => 0.90, 'floor_to_building' => 0.80]

ROOM_DFS = [

'server_room'         => 1.00,   'operating_theater'   => 1.00,

'laboratory'          => 0.90,   'classroom'           => 0.85,

'lecture_hall'        => 0.85,   'workshop'            => 0.85,

'retail_floor'        => 0.85,   'office_open'         => 0.80,

'gym_sports'          => 0.80,   'kitchen_commercial'  => 0.75,

'office_private'      => 0.75,   'prayer_hall'         => 0.75,

'reception_lobby'     => 0.70,   'meeting_room'        => 0.70,

'corridor'            => 0.60,   'living_room'         => 0.60,

'kitchen_residential' => 0.55,   'hotel_room'          => 0.50,

'bedroom'             => 0.45,   'warehouse_storage'   => 0.30,

'bathroom'            => 0.25,

]

DEFAULT_ROOM_DF = 0.80

#### Methods

buildingDfs(?string $type): array — returns ['room_to_floor' => float, 'floor_to_building' => float]; falls back to DEFAULT_BUILDING_DFS if type is null or unknown.

roomDf(?string $type): float — returns the room coincidence factor; falls back to 0.80.

Standard: IEC 60364-8-1 Table B.1 / CIBSE Guide C Table 4.5

### 4.2 SocketDemandService

File: backend/app/Services/SocketDemandService.php

Purpose: Calculates estimated socket outlet demand using a tiered demand factor model.

#### Constant

OUTLET_VA = 200 — assumed load per outlet outlet (VA)

#### Core Method: applyFactors(int $n): float

Demand(n) = min(n, 10) × 200 × 1.00

+ min(max(n-10, 0), 10) × 200 × 0.75

+ max(n-20, 0) × 200 × 0.40

| Outlets | Factor | Per outlet VA |
| --- | --- | --- |
| 1–10 | 100% | 200 VA |
| 11–20 | 75% | 150 VA |
| 21+ | 40% | 80 VA |

#### Methods

roomResult(Room $room) — sums room.sockets.quantity, returns {outlets, connected_va, demand_va}

floorResult(Floor $floor) — sums room sockets + own floor sockets, applies applyFactors(), returns same structure

buildingResult(Building $building) — bulk-queries all floors (2 SQL queries), computes per-floor demand, applies coincidenceFactor(), returns {outlets, sum_floor_demand_va, coincidence_factor, demand_va, connected_va}

projectResult(Project $project) — bulk-queries all buildings (6 SQL queries), aggregates building demands

#### coincidenceFactor(float $demandVA): float

< 50 kVA   → 1.00

50–250 kVA → 0.92

> 250 kVA  → 0.85

### 4.3 SolarIrradianceService

File: backend/app/Services/SolarIrradianceService.php

Purpose: Returns 24-element hourly solar output array in Watts. Primary path: NASA POWER satellite API. Fallback: static PSH lookup + sinusoidal model.

#### Constants

| Constant | Value | Meaning |
| --- | --- | --- |
| ROOF_COVERAGE_RATIO | 0.17 | 17% of roof area usable for panels |
| STC_IRRADIANCE_W | 1000.0 | W/m² at Standard Test Conditions |
| CAPACITY_ESTIMATE_PR | 0.75 | Conservative PR for rough capacity sizing |
| PERFORMANCE_RATIO | 0.80 | PR used in hourly profile computation |
| NASA_API_BASE_URL | https://power.larc.nasa.gov/api/temporal/hourly/point | NASA POWER endpoint |
| NASA_TIMEOUT_SEC | 10 | HTTP timeout seconds |
| NASA_NULL_VALUE | -999 | NASA's null/missing data sentinel |
| CACHE_DAYS | 30 | Cache duration (historical data never changes) |
| REPRESENTATIVE_DAY | 15 | 15th of month used as monthly midpoint |

#### PSH Table (Peak Sun Hours kWh/m²/day)

Latitude |  Jan  Feb  Mar  Apr  May  Jun  Jul  Aug  Sep  Oct  Nov  Dec

0°    |  5.5  5.5  5.5  5.5  5.5  5.5  5.5  5.5  5.5  5.5  5.5  5.5

10°    |  4.9  5.3  5.7  6.1  6.2  6.1  6.1  6.1  5.7  5.3  4.9  4.7

20°    |  4.1  4.8  5.7  6.5  6.9  7.0  6.9  6.6  5.8  4.9  4.0  3.7

30°    |  3.2  4.1  5.4  6.6  7.3  7.7  7.5  6.9  5.8  4.6  3.2  2.8

40°    |  2.0  3.1  4.8  6.3  7.5  8.0  7.7  6.7  5.3  3.8  2.2  1.7

50°    |  0.8  1.9  3.8  5.7  7.3  8.0  7.5  6.1  4.3  2.7  1.0  0.5

60°    |  0.0  0.9  2.7  5.0  7.0  8.0  7.2  5.2  3.1  1.4  0.1  0.0

Source: NASA POWER ALLSKY_SFC_SW_DWN monthly averages, TMY 1991-2020

#### Methods

`estimateCapacityW(float $areaM2): float` — static

= areaM2 × 0.17 × 1000 × 0.75

`getHourlyOutputWatts(?float $lat, ?float $lng, int $month, float $panelCapacityKw, float $PR=0.80, int $day=15): array`

Calls fetchNasaHourlyGhi() — if successful (24 values), uses NASA path

Fallback to hourlyProfile() sinusoidal model

Tracks $dataSource: 'nasa_power' | 'static_lookup' | 'nasa_fallback'

`fetchNasaHourlyGhi(?float $lat, ?float $lng, int $month, int $day): array`

Rounds lat/lng to 4dp (~11m) for stable cache keys

Cache key: nasa_ghi_{lat}_{lng}_{month}_{day}

Fetches ALLSKY_SFC_SW_DWN for year 2023, day 15 of month

Replaces -999 (NASA null) with 0.0

Guard: if all-zero for non-polar location → fallback (avoids caching bad data)

Cache: 30 days via Laravel Cache

`hourlyProfile(float $lat, float $lng, int $month, float $capacityW): array` — static fallback

Formula:

PSH = interpolatePsh(lat, month)

daylight = sunset - sunrise

peakW = capacityW × PSH × PR × π / (2 × daylight)

For each hour h:

t = (h + 0.5 - sunrise) / daylight  [0..1]

output(h) = peakW × sin(π × t)

Handles polar night (output = 0) and midnight sun (flat distribution).

`sunriseSunset(float $lat, int $month): array` — Spencer's equation

DOY = cumulativeDaysBeforeMonth + 15

δ   = 23.45° × sin(360° / 365 × (DOY - 81))     [solar declination]

H_A = arccos(-tan(lat) × tan(δ))                  [hour angle in degrees]

sunrise = 12.0 - H_A / 15

sunset  = 12.0 + H_A / 15

`interpolatePsh(float $lat, int $month): float` — linear interpolation

Takes abs(lat), clamps to 60°

Southern hemisphere: shifts month index by 6 (season flip)

Linearly interpolates between the two bounding 10° latitude bands

### 4.4 BatteryChemistryService

File: backend/app/Services/BatteryChemistryService.php

Purpose: Static registry of battery chemistry performance parameters.

| Chemistry | DoD | RTE | C-charge | C-discharge | Cycles | Life | Degradation/yr |
| --- | --- | --- | --- | --- | --- | --- | --- |
| lead_acid_flooded | 0.50 | 0.80 | 0.10 | 0.20 | 500 | 5yr | 5.0% |
| lead_acid_agm | 0.50 | 0.85 | 0.20 | 0.30 | 700 | 7yr | 4.0% |
| lead_acid_gel | 0.50 | 0.85 | 0.15 | 0.25 | 800 | 8yr | 3.5% |
| lithium_lfp | 0.90 | 0.95 | 0.50 | 1.00 | 4000 | 15yr | 2.0% |
| lithium_nmc | 0.80 | 0.93 | 0.50 | 1.00 | 2500 | 10yr | 2.5% |

Methods: all(): array, getDefaults(string $chemistry): ?array, isValid(string $chemistry): bool

### 4.5 SourceDispatchService

File: backend/app/Services/SourceDispatchService.php

Purpose: Runs a greedy per-hour priority dispatch for all 24 hours of a day. Decides how much each source contributes to meeting load.

#### Constants

GEN_OPTIMAL_MAX_LOAD = 0.85 — generator capped at 85% when opportunistically charging batteries (prevents exceeding efficient operating band)

INV_EFF = 0.95 — AC→DC inverter/rectifier efficiency for generator→battery charging path

#### dispatch() — public entry point

Routes to dispatchOptimized() (batteries present) or dispatchBasic() (no batteries).

#### dispatchBasic() — 4-step per hour (no batteries)

For each hour h (0..23):

demand = loadW[h]

1. Solar: solarUsed = min(solarW[h], demand);  demand -= solarUsed

2. Utility: utilityUsed = min(utilCapW, demand); demand -= utilityUsed

3. Generator: genUsed = min(genCapW, demand);  demand -= genUsed

4. Unmet = max(0, demand)

#### dispatchOptimized() — 7-step per hour (with batteries)

For each hour h:

STEP 1: Each solar system charges its paired batteries proportionally to their headroom

share = systemOutputW × (battery_headroom / total_paired_headroom)

battery.current += (actual_charged / 1000) × η_one_way

STEP 2: Remaining solar covers load directly

solarUsed[h] = min(sharedSolar, demand)

surplus = sharedSolar - solarUsed[h]

STEP 3: Surplus solar charges unpaired batteries (pool)

Each unpaired battery gets proportional share of surplus

STEP 4: All batteries discharge to cover remaining demand

maxDischW = min(Σ disch_kw × 1000, Σ current_kWh × 1000)

Capped by inverter rating per battery (inv_cap_w)

battery.current -= (actual_discharge / 1000) / η_one_way

STEP 5: Utility covers remaining

utilityUsed[h] = min(utilCapW, remaining)

STEP 6: Generator covers remaining

genUsed[h] = min(genCapW, remaining)

STEP 7: Opportunistic generator charging

IF genUsed[h] > 0 AND genCapW > 0:

spare = genCapW × 0.85 - genUsed[h]

IF spare > 0: charge batteries with spare × INV_EFF × η_one_way

genUsed[h] += AC_drawn_for_charging  (generator runs harder)

#### Stats computed

solar_self_consumption = (solar_used_kWh + battery_charged_solar_kWh) / solar_generated_kWh × 100

generator_efficiency_avg = average loading % across hours generator ran

### 4.6 CostSignalService

File: backend/app/Services/CostSignalService.php

Purpose: Computes a 24-element array of marginal cost ($/kWh) for each hour. Answers: "What does the next kW cost this hour?"

#### Constants

BATTERY_DEGRADATION_COST = 0.01 $/kWh — near-zero cost for stored energy (only cell wear)

UNMET_COST = 999.0 — sentinel for hours with no source available

#### Per-hour logic (highest priority to lowest):

IF solar_surplus[h] > 0:          cost = 0.00  (free energy)

ELIF battery_remaining_kw[h] > 0: cost = 0.01  (stored solar)

ELIF utility available:

IF peak hour:                  cost = peak_tariff

ELSE:                          cost = base_tariff

ELIF generator available:          cost = marginal_gen_cost

ELSE:                              cost = 999.0

#### Hour classification

free_hours: cost == 0.0

cheap_hours: 0.0 < cost < baseline

expensive_hours: cost > baseline

(normal: cost == baseline — not listed)

### 4.7 FinancialAnalysisService

File: backend/app/Services/FinancialAnalysisService.php

Purpose: Full financial model: annual costs with and without solar, savings, payback, LCOE, 25-year projection.

#### Constants

PANEL_DEGRADATION = 0.005 (0.5%/year — industry standard crystalline silicon)

PROJECTION_YEARS = 25

DF_PROJECT = 0.7

DEFAULT_WORK_DAYS = ['monday','tuesday','wednesday','thursday','friday']

#### 7-Step analyzeProject() Method

Step 1: Build 24-hour load profile + solar profile for representative Monday in requested month

Step 2: Compute weighted average tariff (off-peak hours × base + peak hours × peak) / 24

Step 3: Run dispatch simulation → get daily kWh per source

Annual solar kWh = daily × 365 (and other sources)

Generator cost = Σₕ F(P_h) × fuel_price × 365 (affine model per hour)

Step 4: Run BASELINE dispatch (no solar, no battery) for comparison

Step 5: Compute annual savings = (baseline_grid + baseline_gen) - (with_grid + with_gen + maintenance)

Step 6: Simple payback = investment / savings; LCOE = (install + maintenance×25) / (solar_kwh × 25)

Step 7: 25-year projection:

runningNet = -totalInvestment

For y = 1..25:

degradFactor = (1 - 0.005)^y

yearNet = annualSavings × degradFactor - batteryReplacementCost[y]

runningNet += yearNet

cumulative[y] = runningNet

if paybackYear == null AND runningNet > 0: paybackYear = y

Battery replacement year calculation:

yearsToEol = max(0, rated_cycle_life / 365 - age_years)

replYear = ceil(yearsToEol)

### 4.8 BackupExporter

File: backend/app/Services/BackupExporter.php

Purpose: Serializes a complete project (all entities, components, sources, members) to a JSON structure for portable backup/restore.

### 4.9 ValidationReferenceService

File: backend/app/Services/ValidationReferenceService.php

Purpose: Returns static reference tables from IEC/NEC standards (cable ampacity, circuit breaker ratings, voltage drop limits, wiring rules) for display on the ValidationPage.

Source: All files in backend/app/Services/

## 5. CONTROLLERS — COMPLETE

### 5.1 TotalPowerController

File: backend/app/Http/Controllers/Api/TotalPowerController.php

| Constant | Value | Meaning |
| --- | --- | --- |
| DF_PROJECT | 0.7 | Project-level diversity factor |
| VOLTAGE_3PHASE_LL | 400 V | IEC line-to-line voltage |
| VOLTAGE_1PHASE | 230 V | IEC line-to-neutral voltage |
| SYSTEM_FREQUENCY | 50 Hz | IEC system frequency |
| TARGET_POWER_FACTOR | 0.95 | PENRA target PF |
| PF_CORRECTION_THRESHOLD | 0.85 | Below this, correction recommended |
| CAPACITOR_BANK_STEP | 0.5 kVAR | Standard manufactured increment |
| INRUSH_MULTIPLIER | 1.25 | NEC Article 430 motor inrush factor |

#### Routes & Methods

`GET /api/projects/{project}/total-power` → project()

Computes full diversified power for entire project (all buildings/floors/rooms). Returns:

{

"total_va": 45230.5, "total": 38450.2,

"max_va": 68000.0, "max_w": 57800.0,

"critical_va": 5000, "critical_w": 4500,

"essential_va": 12000, "essential_w": 10200,

"normal_va": 28230, "normal_w": 23750,

"socket_demand_va": 3500, "socket_connected_va": 7000,

"inrush_applied": true,

"inrush_component": { "name": "Elevator", "per_unit_va": 5000, ... },

"system_power_factor": 0.850,

"pf_correction_recommended": false,

"total_kvar": 23.8, "max_kvar": 35.2,

"capacitor_bank_kvar": null, "capacitor_bank_uf": null,

"current_before_correction_a": 65.4,

"battery_storage": { ... },

"solar_computed": 12750.0

}

`GET /api/buildings/{building}/total-power` → building()

Same structure but scoped to one building.

`GET /api/floors/{floor}/total-power` → floor()

Scoped to one floor.

`GET /api/rooms/{room}/total-power` → room()

Scoped to one room.

#### Key Internal Methods

`sumPowerWithGroups($query, $entityKey, $withPriority, $df)`

Fetches all components, applies group-max deduplication

Applies diversity factor (callable per-entity or flat float)

Critical loads: DF always 1.0

Returns: {va, w, q, max_va, max_w, max_q, critical_va, critical_w, ...} (12 fields)

`reactivePowerFields(...)`

Computes capacitor bank if PF < 0.85:

Q_cap = Q_total - P × tan(arccos(0.95))

bank_kVAR = ceil(Q_cap_kVAR / 0.5) × 0.5

C_per_phase = (bank_kVAR×1000 / 3) / (2π × 50 × 400²) × 10⁶  [μF, delta config]

`applyInrush(?array $motor, &$maxW, &$maxQ)`

Adds 25% extra to max vectors for the largest motor found across all levels.

### 5.2 PhaseBalanceController

File: backend/app/Http/Controllers/Api/PhaseBalanceController.php

Constants: VOLT=230, WARN_PCT=10, CRIT_PCT=20, SOCKET_ASSUMED_PF=0.95

Phase angles: A=0°, B=120°, C=240°

| Route | Method | Purpose |
| --- | --- | --- |
| GET /api/projects/{project}/phase-balance | project() | Balance report for all buildings |
| GET /api/buildings/{building}/phase-balance | building() | Balance + optimal for one building |
| GET /api/floors/{floor}/phase-balance | floor() | Simple greedy for floor |
| PUT /api/rooms/{room}/phase-balance/assign | assignRoom() | Save phase A/B/C to DB for room |
| POST /api/buildings/{building}/phase-balance/apply-optimal | applyOptimalBuilding() | Persist optimal assignment |

Core `buildingReport()` algorithm:

Collect all 1-phase loads grouped as "blocks" (room block, floor-own block, socket block)

Sort blocks descending by VA

Greedy: assign each block to the phase with minimum current VA

Compute phasor currents per phase (accounts for PF/lag, not just VA magnitude)

Compute neutral current = phasor sum of all three phase currents

Imbalance formula:

I_A = VA_A/230, I_B = VA_B/230, I_C = VA_C/230

avg = (I_A + I_B + I_C) / 3

imbalance% = (max - min) / avg × 100

Phasor neutral current:

For each phase X (angle θ_X):

For each load: real += (VA/230) × cos(θ_X - arccos(PF))

imag += (VA/230) × sin(θ_X - arccos(PF))

I_N = sqrt((Σ real)² + (Σ imag)²)

### 5.3 ProjectController

File: backend/app/Http/Controllers/Api/ProjectController.php

| Route | Method | Notes |
| --- | --- | --- |
| GET /api/projects | index() | User's projects with total_power and total_kw |
| POST /api/projects | store() | Create project |
| GET /api/projects/{project} | show() | Single project |
| PUT /api/projects/{project} | update() | Update project |
| DELETE /api/projects/{project} | destroy() | Delete project |
| POST /api/projects/{project}/optimize-shiftable | optimizeShiftable() | Run load scheduler |
| GET /api/projects/{project}/defense-summary | defenseSummary() | Stats for DefensePrepPage |

`optimizeShiftable()` — detailed:

Validate: month (1–12), components array with {id, model_type}

For each component: validate required_run_hours > 0 and fits in window

Build cost signal via buildCostSignal() (tariff + solar layers)

Call pickBestIntervals() to find cheapest window

Improvement guard: compare new cost vs best window in current schedule

Only save if new_cost < current_best_cost - 0.0001

Return {data: [{id, intervals, savings, savings_percent}], optimized_count, errors}

`buildCostSignal()` — combined signal:

solarFrac = solarW[h] / max(solarW)

IF monetary cost exists:

signal[h] = base_cost × (1.0 - 0.9 × solarFrac)

ELSE:

signal[h] = 1.0 - solarFrac

`pickBestIntervals()` — two paths:

maxSplits=0: exhaustive window search O((W-R)×R) where W=window width, R=run_hours

maxSplits>0: sort hours by cost, pick cheapest R hours, group consecutive

### 5.4 ScheduleController

File: backend/app/Http/Controllers/Api/ScheduleController.php

`GET /api/projects/{project}/schedule?month=&day_type=&mode=`

Returns 24-hour dispatch with load profile:

{

"hours": [0..23],

"load_profile_w": [1200, 1200, ...],

"solar_used": [...], "battery_discharged": [...],

"utility_used": [...], "generator_used": [...],

"unmet": [...],

"solar_generated": [...],

"battery_charged": [...],

"battery_soc_trace": [...],

"stats": { "solar_kwh": 12.5, "utility_kwh": 45.2, ... },

"source_capacities": { "solar_kw": 15.0, "battery_kwh": 20.0, ... }

}

### 5.5 LoadProfileController

`GET /api/projects/{project}/load-profile?month=&day_type=`

Returns hourly W array for the project using diversity factors:

{

"hours": [0..23],

"profile": [1500, 1500, 2000, ...],

"peak_w": 8500,

"peak_hour": 14,

"total_kwh": 95.2

}

### 5.6 FinancialController

`GET /api/projects/{project}/financial-analysis?month=`

Calls FinancialAnalysisService::analyzeProject(). Returns:

{

"annual_energy": { "solar_kwh": 4500, "grid_kwh": 12000, "generator_kwh": 0, ... },

"annual_costs": { "grid_cost": 1800, "generator_cost": 0, "total_with_solar": 2000, "total_without_solar": 3200 },

"savings": { "annual_savings": 1200, "savings_percent": 37.5 },

"investment": { "solar_installation": 8000, "battery_purchase": 3000, "total_investment": 11000 },

"payback": { "simple_payback_years": 9.2, "lcoe_solar_per_kwh": 0.0611 },

"projection_25yr": { "cumulative_net_by_year": [...], "payback_year": 9, "total_25yr_benefit": 19500 },

"currency_symbol": "$"

}

### 5.7 CostSignalController

`GET /api/projects/{project}/cost-signal?month=`

Calls full dispatch first, then CostSignalService. Returns:

{

"cost_signal": [0.12, 0.12, 0.12, ..., 0.0, 0.0, 0.12],

"solar_surplus_kw": [0, 0, ..., 2.3, 4.1, ...],

"free_hours": [10, 11, 12, 13],

"cheap_hours": [9, 14],

"expensive_hours": [17, 18, 19],

"currency_symbol": "$",

"meta": { "utility_tariff_per_kwh": 0.12, "peak_tariff_per_kwh": null, ... }

}

### 5.8 ValidationController

`GET /api/validation/reference`

Returns structured reference data from ValidationReferenceService: cable tables, breaker ratings, voltage drop limits, etc.

### 5.9 NavigationController

`GET /api/navigation/{type}/{id}` (type: project/building/floor/room)

Returns breadcrumb array: [{type, id, name}, ...] from root to current entity.

### 5.10 Google OAuth (GoogleController)

File: backend/app/Http/Controllers/Auth/GoogleController.php

| Route | Method | Purpose |
| --- | --- | --- |
| GET /auth/google | redirect() | Redirect to Google consent screen |
| GET /auth/google/callback | callback() | Handle OAuth callback, create/update user, issue Sanctum token |

Flow:

redirect() → Socialite::driver('google')->redirect()

callback() → Socialite::driver('google')->user()

User::updateOrCreate(['email' => $googleUser->email], [...]) with google_id, avatar

$user->createToken('google-token')->plainTextToken

Redirect to frontend /auth/callback?token=...

### 5.11 AdminController

Provides admin dashboard data. Routes behind AdminMiddleware:

| Route | Returns |
| --- | --- |
| GET /api/admin/users | All users with project counts |
| GET /api/admin/stats | Total users, projects, components, backups |

### 5.12 CRUD Controllers (standard patterns)

Each of the following follows standard Laravel CRUD (index/store/show/update/destroy):

| Controller | Resource | Notes |
| --- | --- | --- |
| BuildingController | /api/projects/{project}/buildings |  |
| FloorController | /api/buildings/{building}/floors |  |
| RoomController | /api/floors/{floor}/rooms |  |
| ProjectComponentController | /api/projects/{project}/components |  |
| BuildingComponentController | /api/buildings/{building}/components |  |
| FloorComponentController | /api/floors/{floor}/components |  |
| RoomComponentController | /api/rooms/{room}/components |  |
| UtilityLineController | /api/{type}/{id}/utility-lines | Polymorphic entity |
| GeneratorLineController | /api/{type}/{id}/generator-lines | Polymorphic entity |
| BatteryController | /api/projects/{project}/batteries | Includes chemistry presets endpoint |
| SolarSystemController | /api/projects/{project}/solar-systems |  |
| SocketController | /api/{type}/{id}/sockets | Polymorphic |
| ComponentTypeController | /api/component-types | Shared catalog |
| ProjectMemberController | /api/projects/{project}/members | Add/update/remove members |
| ProjectBackupController | /api/projects/{project}/backup | Export JSON; import JSON |
| ServerBackupController | /api/projects/{project}/server-backups | Create/download/delete |
| UserController | /api/user | Current user profile |

Source: All controller files in backend/app/Http/Controllers/

## 6. MODELS — COMPLETE

### User

$fillable = ['name', 'email', 'password', 'google_id', 'google_token', 'google_refresh_token', 'avatar', 'is_admin']

$hidden   = ['password', 'remember_token']

$casts    = ['email_verified_at' => 'datetime', 'password' => 'hashed', 'is_admin' => 'boolean']

Relationships:

hasMany  → Project (as owner)

belongsToMany → Project (via ProjectUser, as member)

### Project

$fillable = [name, description, area, voltage, frequency, currency_symbol, solar_source,

existing_solar_power, generator_source, location_lat, location_lng,

work_days, working_season_intervals, auto_backup]

$casts    = [work_days => array, working_season_intervals => array, auto_backup => boolean,

generator_source => boolean, location_lat => decimal:7, location_lng => decimal:7]

Relationships:

belongsTo → User (owner)

hasMany   → Building, ProjectComponent, ProjectUser

belongsToMany → User (via ProjectUser)

morphMany → UtilityLine (as utilizable), GeneratorLine (as generable), Socket (as socketable)

hasMany   → Battery, SolarSystem, ServerBackup

Methods:

userRole($userId): ?string — returns 'admin'|'main'|'viewer'|null for the given user

### Building

$fillable = [project_id, name, area, type, existing_solar_power, work_days, working_season_intervals]

$casts    = [work_days => array, working_season_intervals => array]

Relationships:

belongsTo → Project

hasMany   → Floor, BuildingComponent

morphMany → UtilityLine, GeneratorLine, Socket

hasManyThrough → Room (via Floor)

### Floor

$fillable = [building_id, name, work_days, working_season_intervals]

Relationships:

belongsTo → Building

hasMany   → Room, FloorComponent

morphMany → Socket

### Room

$fillable = [floor_id, name, type, work_days, working_season_intervals]

Relationships:

belongsTo → Floor

hasMany   → RoomComponent

morphMany → Socket

### ComponentType

$fillable = [name, default_power, default_power_factor, is_motor]

$casts    = [is_motor => boolean, default_power => decimal:2, default_power_factor => decimal:3]

Relationships:

hasMany → ProjectComponent, BuildingComponent, FloorComponent, RoomComponent

### Component Models (ProjectComponent, BuildingComponent, FloorComponent, RoomComponent)

All four share the same pattern:

$fillable = [component_type_id, component_name, power, power_factor, quantity, phases, phase,

priority, group_name, is_motor, usage_season, usage_day_type, usage_time_intervals,

load_flexibility, required_run_hours, earliest_start_hour, latest_end_hour, max_interruptions]

$casts    = [usage_time_intervals => array, is_motor => boolean]

Relationships:

belongsTo → ComponentType

belongsTo → Project/Building/Floor/Room (parent entity)

### GeneratorLine

$fillable = [name, power, phases, fuel_cost_per_liter, fuel_consumption_lph, no_load_fuel_lph, min_load_pct, optimal_load_pct]

$casts    = [fuel_cost_per_liter => float, fuel_consumption_lph => float, no_load_fuel_lph => float, ...]

$appends  = ['cost_per_kwh']   // computed accessor

Computed methods:

getCostPerKwhAttribute()    → fuel_cost × lph / rated_kW  (flat-rate, at 100% load)

fuelAtLoadKw(float $kW)     → F₀ + (F_rated - F₀) × kW/rated_kW  (ISO 8528 affine)

costPerKwhAtLoad(float $kW) → fuel_cost × fuelAtLoadKw(kW) / kW

marginalCostPerKwh()        → fuel_cost × (F_rated - F₀) / rated_kW  (dC/dP)

generable()                 → morphTo() (Project or Building)

### Battery

$fillable = [project_id, solar_system_id, name, chemistry, nominal_capacity_kwh,

depth_of_discharge, usable_capacity_kwh, round_trip_efficiency,

max_charge_power_kw, max_discharge_power_kw, current_soc,

rated_cycle_life, age_years, is_active, purchase_cost, replacement_cost, annual_maintenance_cost]

Computed (via accessor or model methods):

current_available_kwh = usable_capacity_kwh × current_soc

health_status         = 'good' / 'degraded' / 'replace'  (based on age vs cycle life)

Relationships:

belongsTo → Project, SolarSystem (nullable)

### SolarSystem

$fillable = [project_id, name, capacity_kw, is_active, installation_cost, annual_maintenance_cost]

Relationships:

belongsTo → Project

hasMany   → Battery (batteries paired to this system)

### Socket

$fillable = [quantity]

$casts    = [quantity => integer]

Relationships:

morphTo → socketable (Project, Building, Floor, Room)

### ProjectUser

$fillable = [project_id, user_id, role]

Relationships:

belongsTo → Project, User

Source: All model files in backend/app/Models/

## 7. COMPLETE API REFERENCE

All routes defined in backend/routes/api.php. All routes under /api/ prefix.

Auth middleware: auth:sanctum unless noted.

| Method | Path | Controller@method | Notes |
| --- | --- | --- | --- |
| POST | /auth/google | GoogleController@redirect | Public |
| GET | /auth/google/callback | GoogleController@callback | Public |
| POST | /auth/admin/login | AdminController@login | Public, admin credentials |
| GET | /api/user | UserController@show | auth:sanctum |
| PUT | /api/user | UserController@update | auth:sanctum |
| GET | /api/projects | ProjectController@index |  |
| POST | /api/projects | ProjectController@store |  |
| GET | /api/projects/{project} | ProjectController@show |  |
| PUT | /api/projects/{project} | ProjectController@update |  |
| DELETE | /api/projects/{project} | ProjectController@destroy |  |
| POST | /api/projects/{project}/optimize-shiftable | ProjectController@optimizeShiftable | admin/main role |
| GET | /api/projects/{project}/defense-summary | ProjectController@defenseSummary |  |
| GET | /api/projects/{project}/total-power | TotalPowerController@project |  |
| GET | /api/projects/{project}/load-profile | LoadProfileController@project |  |
| GET | /api/projects/{project}/schedule | ScheduleController@project |  |
| GET | /api/projects/{project}/phase-balance | PhaseBalanceController@project |  |
| GET | /api/projects/{project}/financial-analysis | FinancialController@analyze |  |
| GET | /api/projects/{project}/cost-signal | CostSignalController@project |  |
| GET | /api/projects/{project}/navigation | NavigationController@project |  |
| GET | /api/projects/{project}/buildings | BuildingController@index |  |
| POST | /api/projects/{project}/buildings | BuildingController@store |  |
| GET | /api/buildings/{building} | BuildingController@show |  |
| PUT | /api/buildings/{building} | BuildingController@update |  |
| DELETE | /api/buildings/{building} | BuildingController@destroy |  |
| GET | /api/buildings/{building}/total-power | TotalPowerController@building |  |
| GET | /api/buildings/{building}/phase-balance | PhaseBalanceController@building |  |
| POST | /api/buildings/{building}/phase-balance/apply-optimal | PhaseBalanceController@applyOptimalBuilding | admin/main |
| GET | /api/buildings/{building}/floors | FloorController@index |  |
| POST | /api/buildings/{building}/floors | FloorController@store |  |
| GET | /api/floors/{floor} | FloorController@show |  |
| PUT | /api/floors/{floor} | FloorController@update |  |
| DELETE | /api/floors/{floor} | FloorController@destroy |  |
| GET | /api/floors/{floor}/total-power | TotalPowerController@floor |  |
| GET | /api/floors/{floor}/phase-balance | PhaseBalanceController@floor |  |
| GET | /api/floors/{floor}/rooms | RoomController@index |  |
| POST | /api/floors/{floor}/rooms | RoomController@store |  |
| GET | /api/rooms/{room} | RoomController@show |  |
| PUT | /api/rooms/{room} | RoomController@update |  |
| DELETE | /api/rooms/{room} | RoomController@destroy |  |
| GET | /api/rooms/{room}/total-power | TotalPowerController@room |  |
| PUT | /api/rooms/{room}/phase-balance/assign | PhaseBalanceController@assignRoom | admin/main |
| GET | /api/projects/{project}/components | ProjectComponentController@index |  |
| POST | /api/projects/{project}/components | ProjectComponentController@store |  |
| PUT | /api/projects/{project}/components/{component} | ProjectComponentController@update |  |
| DELETE | /api/projects/{project}/components/{component} | ProjectComponentController@destroy |  |
| GET | /api/buildings/{building}/components | BuildingComponentController@index |  |
| POST | /api/buildings/{building}/components | BuildingComponentController@store |  |
| PUT | /api/buildings/{building}/components/{component} | BuildingComponentController@update |  |
| DELETE | /api/buildings/{building}/components/{component} | BuildingComponentController@destroy |  |
| GET | /api/floors/{floor}/components | FloorComponentController@index |  |
| POST | /api/floors/{floor}/components | FloorComponentController@store |  |
| PUT/DELETE | /api/floors/{floor}/components/{component} | FloorComponentController@... |  |
| GET | /api/rooms/{room}/components | RoomComponentController@index |  |
| POST | /api/rooms/{room}/components | RoomComponentController@store |  |
| PUT/DELETE | /api/rooms/{room}/components/{component} | RoomComponentController@... |  |
| GET/POST/PUT/DELETE | /api/.../utility-lines/... | UtilityLineController | Polymorphic |
| GET/POST/PUT/DELETE | /api/.../generator-lines/... | GeneratorLineController | Polymorphic |
| GET/POST/PUT/DELETE | /api/.../sockets/... | SocketController | Polymorphic |
| GET/POST/PUT/DELETE | /api/projects/{project}/batteries/... | BatteryController |  |
| GET | /api/batteries/chemistry-presets | BatteryController@presets |  |
| GET/POST/PUT/DELETE | /api/projects/{project}/solar-systems/... | SolarSystemController |  |
| GET/POST/PUT/DELETE | /api/component-types/... | ComponentTypeController |  |
| GET/POST/PUT/DELETE | /api/projects/{project}/members/... | ProjectMemberController |  |
| POST | /api/projects/{project}/backup/export | ProjectBackupController@export |  |
| POST | /api/projects/{project}/backup/import | ProjectBackupController@import |  |
| GET/POST/DELETE | /api/projects/{project}/server-backups/... | ServerBackupController |  |
| GET | /api/admin/users | AdminController@users | AdminMiddleware |
| GET | /api/admin/stats | AdminController@stats | AdminMiddleware |
| GET | /api/validation/reference | ValidationController@reference |  |

Source: backend/routes/api.php, all controller files

## 8. FORM REQUEST VALIDATION RULES

### StoreProjectRequest / UpdateProjectRequest

name:         required|string|max:255

description:  nullable|string

area:         nullable|numeric|min:0

voltage:      nullable|string|max:50

frequency:    nullable|integer|in:50,60

currency_symbol: nullable|string|max:10

solar_source: nullable|in:max,existing

location_lat: nullable|numeric|between:-90,90

location_lng: nullable|numeric|between:-180,180

work_days:    nullable|array

### StoreBuildingRequest / UpdateBuildingRequest

name:  required|string|max:255

area:  nullable|numeric|min:0

type:  nullable|string|in:[all building type keys]

### StoreComponentRequest / UpdateComponentRequest

component_type_id:    required|integer|exists:component_types,id

power:                required|numeric|min:0

power_factor:         nullable|numeric|between:0.01,1

quantity:             nullable|integer|min:1|max:1000

phases:               nullable|in:1phase,3phase

priority:             nullable|in:critical,essential,normal

group_name:           nullable|string|max:255

usage_season:         nullable|in:all,summer,winter,spring,autumn

usage_day_type:       nullable|in:all,weekday,weekend

usage_time_intervals: nullable|array

load_flexibility:     nullable|in:fixed,shiftable,curtailable

required_run_hours:   nullable|integer|min:1|max:24

earliest_start_hour:  nullable|integer|min:0|max:23

latest_end_hour:      nullable|integer|min:1|max:24

max_interruptions:    nullable|integer|min:0

### StoreBatteryRequest / UpdateBatteryRequest

name:                   required|string|max:255

chemistry:              required|string|in:[5 chemistry keys]

nominal_capacity_kwh:   required|numeric|min:0.1

depth_of_discharge:     required|numeric|between:0.1,1

round_trip_efficiency:  required|numeric|between:0.5,1

max_charge_power_kw:    required|numeric|min:0.1

max_discharge_power_kw: required|numeric|min:0.1

current_soc:            nullable|numeric|between:0,1

rated_cycle_life:       required|integer|min:10

solar_system_id:        nullable|integer|exists:solar_systems,id

purchase_cost:          nullable|numeric|min:0

replacement_cost:       nullable|numeric|min:0

### StoreUtilityLineRequest / UpdateUtilityLineRequest

name:                  nullable|string

power:                 nullable|numeric|min:0

phases:                nullable|in:1phase,3phase

tariff_per_kwh:        nullable|numeric|min:0

peak_tariff_per_kwh:   nullable|numeric|min:0

peak_hours_start:      nullable|integer|min:0|max:23

peak_hours_end:        nullable|integer|min:1|max:24

### StoreGeneratorLineRequest / UpdateGeneratorLineRequest

name:                  nullable|string

power:                 nullable|numeric|min:0

phases:                nullable|in:1phase,3phase

fuel_cost_per_liter:   nullable|numeric|min:0

fuel_consumption_lph:  nullable|numeric|min:0

no_load_fuel_lph:      nullable|numeric|min:0

min_load_pct:          nullable|integer|min:0|max:100

optimal_load_pct:      nullable|integer|min:0|max:100

Source: All files in backend/app/Http/Requests/

## 9. FRONTEND PAGES — COMPLETE

### LoginPage (/login)

Shows Google OAuth button

Calls: GET /auth/google (redirects to Google)

Redirects logged-in users to /dashboard (PublicRoute guard)

### AuthCallbackPage (/auth/callback)

Reads ?token= from URL on return from Google OAuth

Calls AuthContext.login(token) to store token and fetch user

Redirects to /dashboard

### DashboardPage (/dashboard)

Lists all user's projects

API: GET /api/projects

Shows total_kw (kW, primary) and total_power (kVA, secondary) per project

Delete project inline

Link to /projects/new for creation

ProjectRow component with stacked kW/kVA display

### CreateProjectPage (/projects/new)

Form: name, description, area, voltage, frequency, currency

API: POST /api/projects

On success: navigates to new project page

### ProjectPage (/projects/:projectId)

Top-level project overview

Shows <PowerBanner> with project-level total power

Shows <PowerSourcesBanner> with solar/battery/grid/generator summary

API: GET /api/projects/:id/total-power

### BuildingPage (/projects/:id/buildings/:bid)

Building detail: components table + power banner

API: GET /api/buildings/:bid/total-power, GET /api/buildings/:bid/components

Shows <EntityComponents>, <EntitySockets>, <PowerBanner>

### FloorPage, NewRoomPage — same pattern for floors and rooms

### LoadSchedulePage (/projects/:id/schedule)

Three tabs: Schedule (base load profile), Optimized (after optimization), Combined

Mode selector: base / optimized

Cost signal bar chart (24 bars color-coded by cost level)

Per-component schedule grid showing intervals

"Optimize" button → POST /api/projects/:id/optimize-shiftable

API: GET /api/projects/:id/schedule, GET /api/projects/:id/cost-signal

On optimize success: switches to 'combined' tab and 'optimized' mode

### PhaseBalancePage (/projects/:id/phase-balance)

Shows Actual vs Optimal phase distribution per building

Doughnut chart of A/B/C percentage

Imbalance percentage with status color

Neutral current display

"Apply Optimal" button → POST /api/buildings/:id/phase-balance/apply-optimal

API: GET /api/projects/:id/phase-balance

### FinancialPage (/projects/:id/financial)

Month selector (1–12)

Energy mix pie chart (solar/grid/generator %)

Cost comparison card: with solar vs without solar (annual)

25-year cumulative net chart (Recharts LineChart)

Payback year highlighted

Investment summary: installation + battery costs

API: GET /api/projects/:id/financial-analysis?month=

### ValidationPage (/validation)

IEC/NEC reference browser

Tables: cable ratings, breaker sizes, voltage drop limits

API: GET /api/validation/reference

Standalone route (not inside ProjectLayout)

### DefensePrepPage (/defense-prep)

Admin-only (checks user.is_admin)

42 accordion Q&A items across 8 categories

Project selector to pull live data

Category filter tabs

"Expand All" / "Print All" buttons

Print CSS with "Power Profile — Defense Preparation — Ahmed Zoher — June 2026" footer

Live data pills (pulsing dot) for questions using real project data

### SingleLineDiagramPage (/projects/:id/single-line)

Auto-generated SVG showing power sources → bus → building panels

SourceNode (solar/battery/grid/generator), BuildingNode, Wire, Breaker SVG components

Download SVG button

Legend strip

Disclaimer text

### AdminLoginPage (/admin/login)

Email + password form

POST /api/auth/admin/login

Issues admin-level Sanctum token

### AdminDashboardPage (/admin/dashboard)

System statistics (total users, projects, components)

User list with project counts

Server backup management

## 10. FRONTEND COMPONENTS — COMPLETE

### Navbar

Top bar with project name, user avatar, logout

"Defense Prep" button (only shown when user.is_admin)

Uses useAuth() context

### PowerBanner

Displays MAX LOAD / OPTIMIZED kVA and kW

Toggle: VA view | W view (default: VA)

PDF export button → calls printPowerReport()

Excel export button → calls exportProjectExcel() (shows spinner during export)

Props: data (power response), title, projectId, capApplied

### PowerSourcesBanner

Shows all configured power sources for the project

Solar: capacity kW + computed output

Battery: total kWh, current SOC, health

Grid: capacity kVA, tariff

Generator: rated kW, fuel cost/kWh

API: GET /api/projects/:id/schedule for live dispatch data

### ProjectSidebar

Navigation links for all project sub-pages

Active state detection via useLocation()

Links: Project Overview, Buildings, Load Schedule, Phase Balance, Financial Analysis, Single-Line Diagram

"Single-Line Diagram" shown with cyan styling

### EntityComponents

Reusable CRUD table for components at any entity level

Props: entityType, entityId

Features: add/edit/delete, inline editing, group_name field, priority selector, PF indicator

Shows <TimeScheduleModal> for scheduling

### EntityScheduleModal

Modal for editing entity work days and season intervals

Props: entity, entityType, onSaved

### EntitySockets

Shows socket outlets for any entity

Displays demand VA and connected VA

Props: entityType, entityId

### TimeScheduleModal

Modal for editing usage_time_intervals on a component

Visual time blocks across 24-hour bar

Also edits: load_flexibility, required_run_hours, earliest/latest window, max_interruptions

### ReactivePowerPanel

Shown when pf_correction_recommended = true

Displays capacitor bank details: kVAR, μF, current reduction %

Color-coded PF status pill

### BackupChoiceModal

Offers "Download JSON" (client-side) or "Save to Server" options

Triggers appropriate backup action on choice

### ProjectMembersModal

List of project members with their roles

Add member by email

Change role (viewer/main/admin) inline

Remove member

### ServerBackupsList

List of server backups for a project

Download and delete buttons per backup

### UserCard

Avatar circle + name text

Used in member lists and admin dashboard

### ErrorBoundary

Wraps each major route in App.jsx

Catches render errors, shows "Something went wrong" with label prop context

### LoadingSpinner

Centered animated spinner

Shown during <Suspense> lazy load of pages

## 11. UTILITY FILES

### contexts/AuthContext.jsx

React context providing: user, isLoading, login(token), logout()

On mount: reads token from localStorage, calls GET /api/user to hydrate user object

login(token): stores to localStorage, sets axios default header, fetches user

logout(): clears localStorage, clears axios header, sets user to null

user object shape: { id, name, email, avatar, is_admin }

### layouts/ProjectLayout.jsx

Wrapper for all /projects/:projectId/* routes

Fetches GET /api/projects/:projectId on mount

Provides project data to all child pages via context or props

Shows <ProjectSidebar> alongside page content

Handles project-not-found → redirect to dashboard

### api/axios.js

Base URL: configured from VITE_API_BASE_URL env or defaults to backend server

Default header: Authorization: Bearer {token} from localStorage

Interceptor: on 401 response → logout() and redirect to /login

Interceptor: adds Accept: application/json to all requests

### App.jsx

All pages lazy-loaded via React.lazy()

ProtectedRoute: redirects to /login if no user

PublicRoute: redirects to /dashboard if already logged in

BrowserRouter with future flags v7_startTransition, v7_relativeSplatPath

<Suspense fallback={<LoadingSpinner />}> wraps all routes

Each route wrapped in <ErrorBoundary label="..."> for isolated error handling

### utils/exportToExcel.js

exportProjectExcel(projectId, projectName, powerData, engineerName)

Sheet 1 — Summary: project metadata, demand summary (max/optimized), PF status, priority breakdown, socket demand, capacitor bank (if applicable)

Sheet 2 — Components: fetches full hierarchy via 6 nested API calls. Columns: Level, Location, Name, Power(W), Qty, PF, TotalW, Phases, Priority, Scheduling, Season, DayType

Sheet 3 — Financial: annual summary rows + 25-year yearly cashflow table (Year, Solar kWh, Savings, Battery Replacement, Net, Cumulative)

Header row style: blue background (#1E3A8A), white bold text

Data rows: alternating white/light-gray (#F9FAFB)

Column widths set explicitly via ws['!cols']

### utils/printPowerReport.js

printPowerReport(data, title, { capApplied, engineerName })

Opens new browser window, writes full HTML with inline CSS, calls window.print()

Sections: letterhead (logo + project name + engineer + date + ref#), 3 summary cards (Max Load / Optimized / Diversity Reduction %), PF status pill (green ≥0.95 / amber ≥0.85 / red <0.85), priority breakdown table, capacitor bank block (if applicable), signature block, standards footer

Auto-generated ref: PP-${Date.now().toString(36).toUpperCase()}-${Math.random().toString(36).substr(2,4).toUpperCase()}

### utils/downloadJson.js

Creates a Blob from JSON string, creates an <a> element with object URL, triggers download

Used by ProjectBackupController export flow

### utils/navContext.js

Navigation context helper for passing project/building/floor/room context between pages without prop drilling

Source: All utility files, frontend/src/contexts/AuthContext.jsx, frontend/src/layouts/ProjectLayout.jsx, frontend/src/App.jsx

## 12. ALL ELECTRICAL FORMULAS

Every formula found in the codebase, with variable definitions, standards, source files, and worked examples.

### 12.1 Apparent Power (Power Triangle)

Formula: S = sqrt(P^2 + Q^2)

Variables:

S = Apparent power (VA)

P = Active power (W)

Q = Reactive power (VAR)

Standard: IEC 60364-8-1

Source: ValidationReferenceService.php, TotalPowerController (via ValidationController.php)

Worked Example:

P = 5,000 W, Q = 1,750 VAR

S = sqrt(5000^2 + 1750^2) = sqrt(25,000,000 + 3,062,500) = sqrt(28,062,500) ≈ 5,297 VA

### 12.2 Active Power from Apparent Power

Formula: P = S × cos(φ) = S × PF

Variables:

P = Active power (W)

S = Apparent power (VA)

PF = Power factor = cos(φ) (dimensionless, 0–1)

Standard: IEC 60364-8-1

Source: TotalPowerController.php, ValidationReferenceService.php

Worked Example:

S = 3,000 VA, PF = 0.85

P = 3,000 × 0.85 = 2,550 W

### 12.3 Reactive Power

Formula: Q = P × tan(arccos(PF))

Variables:

Q = Reactive power (VAR)

P = Active power (W)

PF = Power factor (dimensionless)

Standard: IEC 60364-8-1

Source: ValidationReferenceService.php, TotalPowerController.php

Worked Example:

P = 2,550 W, PF = 0.85 → φ = arccos(0.85) = 31.79° → tan(31.79°) = 0.6197

Q = 2,550 × 0.6197 ≈ 1,580 VAR

### 12.4 System Power Factor

Formula: PF_sys = P_total / S_total

Variables:

PF_sys = System power factor (dimensionless)

P_total = Total active power (W)

S_total = Total apparent power (VA)

Standard: PENRA / IEC 60364-8-1

Source: ValidationController.php line 231: $pf_sys = $total_va > 0 ? round($total_w / $total_va, 3) : 1.0

Worked Example:

P = 8,200 W, S = 9,100 VA

PF_sys = 8,200 / 9,100 ≈ 0.901

### 12.5 Diversity Cascade (Full Chain)

Formula: DF_effective = DF_room_type × DF_room_to_floor × DF_floor_to_building × DF_project

Variables:

DF_room_type = Room coincidence factor (from CIBSE table, e.g. 0.80 for office_open)

DF_room_to_floor = Building-type diversity factor, room level (e.g. 0.85 for office)

DF_floor_to_building = Building-type diversity factor, floor level (e.g. 0.80 for office)

DF_project = Project-level diversity factor = 0.70 (IEC 60364-8-1 constant)

Standard: IEC 60364-8-1, BS 7671, CIBSE Guide C

Source: DiversityFactorService.php, ScheduleController.php line 243, FinancialAnalysisService.php line 295

Worked Example (open-plan office room):

DF_room_type (office_open) = 0.80

DF_room_to_floor (office) = 0.85

DF_floor_to_building (office) = 0.80

DF_project = 0.70

DF_effective = 0.80 × 0.85 × 0.80 × 0.70 = 0.3808

A 5,000 VA air conditioner contributes 5,000 × 0.3808 = 1,904 VA to the diversified total.

### 12.6 Socket Tiered Demand

Formula:

First 10 outlets: demand = n × 200 VA × 1.00

Next 10 outlets (11–20): demand += (n-10) × 200 VA × 0.75

Remaining (>20): demand += (n-20) × 200 VA × 0.40

Variables:

n = Total number of socket outlets

200 VA = Standard outlet rating per IEC/BS 7671

Standard: BS 7671 / IEC demand factor for socket circuits

Source: SocketDemandService.php lines 19–23

Worked Example:

n = 28 outlets

Tier 1 (10): 10 × 200 × 1.00 = 2,000 VA

Tier 2 (10): 10 × 200 × 0.75 = 1,500 VA

Tier 3 (8): 8 × 200 × 0.40 = 640 VA

Total demand = 4,140 VA

### 12.7 Socket Coincidence Factor

Formula:

Raw demand < 50 kVA: CF = 1.00

Raw demand 50–250 kVA: CF = 0.92

Raw demand > 250 kVA: CF = 0.85

Variables:

CF = Coincidence factor (dimensionless)

Raw demand = Sum of floor-level socket demands (VA)

Standard: IEC 60364 / CIBSE Guide C building-level demand

Source: SocketDemandService.php lines 213–219

Worked Example:

Floor 1: 20 outlets → 3,500 VA; Floor 2: 8 outlets → 1,600 VA

Raw = 5,100 VA = 5.1 kVA → CF = 1.00

Final building socket demand = 5,100 × 1.00 = 5,100 VA

### 12.8 Capacitor Bank Size

Formula: Q_cap = Q_sys - P × tan(arccos(PF_target))

Then round up to next 0.5 kVAR step.

Variables:

Q_cap = Required reactive compensation (VAR)

Q_sys = Current system reactive power (VAR)

P = System active power (W)

PF_target = 0.95 (PENRA target)

CAP_STEP_KVAR = 0.5 kVAR (standard bank increment)

Standard: IEC 60831 (capacitor banks), PENRA PF requirements

Source: ValidationController.php lines 238–245, ValidationReferenceService.php lines 113–118

Worked Example:

P = 8,200 W, Q = 3,500 VAR, PF_target = 0.95

Q_target = 8,200 × tan(arccos(0.95)) = 8,200 × 0.3287 = 2,695 VAR

Q_cap = 3,500 - 2,695 = 805 VAR = 0.805 kVAR

Rounded up to next 0.5 step: cap_bank_kvar = 1.0 kVAR

### 12.9 Capacitor Value (Delta Connection)

Formula: C_phase_μF = (Q_cap_per_phase / (2π × f × V_LL^2)) × 1,000,000

Variables:

C_phase_μF = Capacitor value per phase (μF)

Q_cap_per_phase = Q_cap_total / 3 (VAR per phase)

f = System frequency = 50 Hz

V_LL = Line-to-line voltage = 400 V

Standard: IEC 60831 (delta-connected 3-phase capacitor banks)

Source: ValidationController.php lines 242–245, ValidationReferenceService.php lines 116–120

Worked Example:

Q_cap = 1.0 kVAR = 1,000 VAR → Q per phase = 333.3 VAR

C = (333.3 / (2π × 50 × 400^2)) × 1e6 = (333.3 / 50,265,482) × 1e6 ≈ 6.63 μF

### 12.10 Inrush 125% Rule (NEC Article 430)

Formula: I_design = I_rated × 1.25

Variables:

I_design = Design current for motor circuit (A) — used for breaker/conductor sizing

I_rated = Rated full-load motor current (A)

1.25 = NEC Article 430 safety factor for continuous motor loads

Standard: NEC Article 430

Source: Referenced in printPowerReport.js standards footer; applied in TotalPowerController motor detection

Worked Example:

Motor rated current = 40 A

Design current = 40 × 1.25 = 50 A → select 50 A breaker minimum

### 12.11 Phase Imbalance Percentage

Formula: Imbalance% = ((I_max - I_min) / I_avg) × 100

Variables:

I_max = Highest phase current magnitude (A)

I_min = Lowest phase current magnitude (A)

I_avg = (I_A + I_B + I_C) / 3 = Average phase current (A)

I_x = VA_x / V_phase = Phase x current approximation (A) for VA-based calculation

Standard: IEC 60034-26 (motor voltage imbalance); NEMA MG-1

Source: PhaseBalanceController.php lines 449–461

Thresholds: < 10% = balanced; 10–20% = warning; > 20% = critical

Worked Example:

Phase A: 1,380 VA → I_A = 1380/230 = 6.0 A

Phase B: 920 VA → I_B = 920/230 = 4.0 A

Phase C: 1,150 VA → I_C = 1150/230 = 5.0 A

I_avg = (6.0 + 4.0 + 5.0) / 3 = 5.0 A

Imbalance = (6.0 - 4.0) / 5.0 × 100 = 40% → CRITICAL

### 12.12 Phase Current Phasor (IEC Phasor Method)

Formula per phase:

|I| = S / V_phase  (current magnitude, A)

φ = arccos(PF)  (lag angle, rad)

θ_I = θ_V - φ  (current angle = voltage angle minus power factor angle)

I_real = |I| × cos(θ_I)

I_imag = |I| × sin(θ_I)

Phase angles: θ_V(A) = 0 rad, θ_V(B) = 2π/3 rad (120°), θ_V(C) = 4π/3 rad (240°)

Variables:

S = Load apparent power on this phase (VA)

V_phase = 230 V (single-phase voltage)

PF = Load power factor

θ_V = Phase voltage angle (rad)

I_real, I_imag = Real and imaginary components of current phasor

Standard: IEC 60909 (short-circuit phasor analysis); positive-sequence ABC convention

Source: PhaseBalanceController.php lines 294–312

Worked Example:

Phase A load: 2,300 VA, PF = 0.85, θ_V = 0

|I| = 2300/230 = 10 A; φ = arccos(0.85) = 31.79° = 0.5548 rad

θ_I = 0 - 0.5548 = -0.5548 rad

I_real = 10 × cos(-0.5548) = 10 × 0.85 = 8.5 A

I_imag = 10 × sin(-0.5548) = 10 × (-0.527) = -5.27 A

### 12.13 Neutral Current (Phasor Sum)

Formula:

I_N_real = I_A_real + I_B_real + I_C_real

I_N_imag = I_A_imag + I_B_imag + I_C_imag

|I_N| = sqrt(I_N_real^2 + I_N_imag^2)

Variables:

I_A_real, I_A_imag = Real and imaginary components of Phase A total current

I_B_real, I_B_imag = Phase B components (at 120° offset)

I_C_real, I_C_imag = Phase C components (at 240° offset)

|I_N| = Neutral current magnitude (A)

Standard: IEC 60909, Kirchhoff's Current Law

Source: PhaseBalanceController.php lines 315–323

Worked Example (perfectly balanced, PF=1):

10 A per phase, θ_V(A)=0, θ_V(B)=120°, θ_V(C)=240°

I_A = (10, 0); I_B = (-5, 8.66); I_C = (-5, -8.66)

Sum: real = 0, imag = 0 → |I_N| = 0 A (perfect balance)

### 12.14 Solar Capacity Estimate from Roof Area

Formula: P_cap (W) = A × 0.17 × 1,000 × 0.75

Variables:

A = Roof/building area (m²)

0.17 = Roof coverage ratio (17% usable for PV panels)

1,000 W/m² = STC irradiance (Standard Test Conditions)

0.75 = Conservative performance ratio for capacity sizing

Standard: IEC 61853 (PV performance), STC per IEC 60904-3

Source: SolarIrradianceService.php lines 33–49: estimateCapacityW()

Worked Example:

Building area = 500 m²

P_cap = 500 × 0.17 × 1000 × 0.75 = 63,750 W ≈ 63.75 kW

### 12.15 PSH-Based Sinusoidal Peak Solar Output

Formula: P_peak_W = P_capacity × PSH × PR × π / (2 × T_daylight)

Where hourly output at time t (hours after sunrise):

P(t) = P_peak_W × sin(π × t / T_daylight)

Variables:

P_capacity = Installed panel capacity (W)

PSH = Peak Sun Hours for latitude/month (kWh/m²/day, from lookup table)

PR = Performance Ratio = 0.80 (inverter + wiring + temperature losses)

T_daylight = Sunset - Sunrise (hours)

t = Hours elapsed since sunrise (0 ≤ t ≤ T_daylight)

Derivation: Peak is set so that the integral of sin(πt/D) from 0 to D equals D×2/π, making total daily energy = P_capacity × PSH × PR.

Standard: NASA POWER methodology; IEC 61724 (PV system performance monitoring)

Source: SolarIrradianceService.php lines 107–122

Worked Example (lat=30°, July, 10 kW system):

PSH = 7.5 kWh/m²/day (from PSH_TABLE at 30°, month=7)

Sunrise ≈ 5.5h, Sunset ≈ 18.5h → T_daylight = 13h

P_peak = 10,000 × 7.5 × 0.80 × π / (2 × 13) = 10,000 × 7.5 × 0.80 × 0.1208 = 7,243 W

At solar noon (t=6.5h): P = 7,243 × sin(π × 6.5/13) = 7,243 × sin(π/2) = 7,243 W

### 12.16 Solar Output from NASA GHI

Formula: P_output (W) = (GHI / 1,000) × P_capacity_W × PR

Variables:

GHI = Global Horizontal Irradiance (W/m²) from NASA POWER API (ALLSKY_SFC_SW_DWN)

1,000 = STC reference irradiance (W/m²)

P_capacity_W = Installed system capacity (W)

PR = Performance Ratio = 0.80

Standard: IEC 61724, NASA POWER satellite data (±3% accuracy)

Source: SolarIrradianceService.php lines 341–350

Worked Example:

GHI at noon = 850 W/m², P_capacity = 20,000 W, PR = 0.80

P_output = (850/1000) × 20,000 × 0.80 = 13,600 W

### 12.17 Sunrise and Sunset (Spencer's Equation)

Formula:

δ = 23.45 × sin(360/365 × (DOY - 81)) degrees  (solar declination)

cos(H_A) = -tan(lat) × tan(δ)

H_A_degrees = arccos(cos(H_A)) in degrees

Sunrise = 12.0 - H_A_degrees / 15.0  (decimal hours)

Sunset = 12.0 + H_A_degrees / 15.0

Variables:

δ = Solar declination (degrees)

DOY = Day of year (15th of each month: Jan=15, Feb=46, Mar=74, ...)

lat = Latitude (degrees, negative = southern hemisphere)

H_A = Hour angle at sunrise/sunset (degrees; divided by 15 to convert to hours)

Special: cos(H_A) > 1 → polar night (no sunrise); cos(H_A) < -1 → midnight sun

Standard: Spencer (1971) solar geometry; ASHRAE

Source: SolarIrradianceService.php lines 131–150

Worked Example (lat=30°N, July 15, DOY=196):

δ = 23.45 × sin(360/365 × (196-81)) = 23.45 × sin(113.4°) = 23.45 × 0.917 = 21.5°

cos(H_A) = -tan(30°) × tan(21.5°) = -0.577 × 0.394 = -0.227

H_A = arccos(-0.227) = 103.1° → H_A/15 = 6.87 hours

Sunrise = 12.0 - 6.87 = 5.13 (≈05:08); Sunset = 12.0 + 6.87 = 18.87 (≈18:52)

### 12.18 PSH Latitude Interpolation

Formula: PSH(lat, month) = PSH_lower + frac × (PSH_upper - PSH_lower)

Where:

lower = floor(abs(lat) / 10) × 10 (lower latitude band, multiples of 10°)

upper = min(lower + 10, 60)

frac = (abs(lat) - lower) / 10.0

Southern hemisphere: flip month index by +6 modulo 12

Variables:

lat = Latitude in degrees

PSH_lower, PSH_upper = PSH values from lookup table at bounding latitude bands

frac = Fractional position between bands (0.0–1.0)

Source: SolarIrradianceService.php lines 169–189

Worked Example (lat=35°N, June):

lower=30, upper=40, frac=(35-30)/10=0.5

PSH_30_June = 7.7; PSH_40_June = 8.0

PSH = 7.7 + 0.5 × (8.0 - 7.7) = 7.7 + 0.15 = 7.85 kWh/m²/day

### 12.19 Battery State of Charge Update

Charging:

SOC_new = SOC_old + (P_charge_W / 1000) × η_one_way / C_usable_kWh

Discharging:

SOC_new = SOC_old - (P_discharge_W / 1000) / η_one_way / C_usable_kWh

Where η_one_way = sqrt(RTE)  (one-way efficiency from round-trip)

Variables:

SOC = State of Charge (0.0–1.0)

P_charge_W = Charging power (W)

P_discharge_W = Discharge power (W)

η_one_way = sqrt(RTE) = one-way efficiency (e.g. sqrt(0.95) = 0.9747 for LFP)

C_usable_kWh = Usable capacity (kWh) = nominal × DoD × age_factor

RTE = Round-trip efficiency (e.g. 0.95)

Standard: IEC 62619, IEEE 1562

Source: SourceDispatchService.php lines 211, 272–273

Worked Example:

C_usable = 10 kWh, RTE = 0.95 → η = sqrt(0.95) = 0.9747

Charge 5,000 W for 1 hour: SOC gain = (5.0 × 0.9747) / 10.0 = 0.4874

### 12.20 Battery Round-Trip Efficiency Loss

Formula: E_loss = E_charged - E_discharged = sum(P_charge × dt) - sum(P_discharge × dt)

Variables:

E_loss = Energy lost in battery (kWh per day)

All power in kWh (W/1000)

Source: SourceDispatchService.php line 421: battery_efficiency_loss_kwh = charged_kwh - discharged_kwh

Worked Example:

Charged: 8 kWh, Discharged: 7.4 kWh → Loss = 0.6 kWh (7.5% loss, consistent with RTE=0.93)

### 12.21 Generator Flat-Rate Cost (Rated Load)

Formula: C_rated = (fuel_price × F_rated) / P_rated_kW

Variables:

C_rated = Cost per kWh at full rated load ($/kWh)

fuel_price = Fuel cost per liter ($/L)

F_rated = Fuel consumption at rated load (L/h)

P_rated_kW = Generator rated power (kW) = power_W / 1000

Standard: ISO 8528-5 (generator fuel consumption)

Source: GeneratorLine.php lines 26–37: getCostPerKwhAttribute()

Worked Example:

fuel_price = $0.90/L, F_rated = 15 L/h, P_rated = 50 kW

C_rated = (0.90 × 15) / 50 = $0.27/kWh

### 12.22 Generator Affine Fuel Model F(P)

Formula: F(P) = F_0 + (F_rated - F_0) × (P / P_rated)

Variables:

F(P) = Fuel consumption at load P (L/h)

F_0 = No-load fuel consumption (L/h); default = 0.30 × F_rated if not set

F_rated = Full-load fuel consumption (L/h)

P = Actual output power (kW)

P_rated = Rated generator power (kW)

Standard: ISO 8528-10, CIBSE Guide L (generator part-load performance)

Source: GeneratorLine.php lines 45–62: fuelAtLoadKw()

Worked Example:

F_0 = 4.5 L/h (30% of 15), F_rated = 15 L/h, P_rated = 50 kW, P = 25 kW

F(25) = 4.5 + (15 - 4.5) × (25/50) = 4.5 + 10.5 × 0.5 = 4.5 + 5.25 = 9.75 L/h

Compare to flat rate: 15 L/h × (25/50) = 7.5 L/h → affine model shows 30% more at half load

### 12.23 Generator Marginal Cost

Formula: MC = fuel_price × (F_rated - F_0) / P_rated

Variables:

MC = Marginal cost ($/kWh) — cost of serving 1 additional kW

fuel_price, F_rated, F_0, P_rated as above

Note: MC < average cost because no-load overhead is already sunk.

Standard: ISO 8528, economic dispatch theory

Source: GeneratorLine.php lines 81–97: marginalCostPerKwh()

Worked Example:

fuel_price = $0.90/L, F_rated = 15 L/h, F_0 = 4.5 L/h, P_rated = 50 kW

MC = 0.90 × (15 - 4.5) / 50 = 0.90 × 10.5 / 50 = $0.189/kWh

vs flat-rate $0.27/kWh → marginal is 30% cheaper

### 12.24 Cost Signal Ladder

Formula (per hour):

If solar_surplus > 0: cost = 0.00 $/kWh

Else if battery_remaining_kw > 0: cost = 0.01 $/kWh

Else if utility available and off-peak: cost = tariff $/kWh

Else if utility available and peak hours: cost = peak_tariff $/kWh

Else if generator available: cost = marginal_cost $/kWh

Else: cost = 999.00 $/kWh (load shedding sentinel)

Variables:

solar_surplus = max(0, solar_generated_kW - load_kW)

battery_remaining_kw = unused discharge headroom from dispatch

tariff = utility base rate ($/kWh)

peak_tariff = utility peak-period rate ($/kWh)

marginal_cost = generator marginal cost ($/kWh)

Source: CostSignalService.php lines 90–129

### 12.25 Solar Cost Discount Formula (Optimizer)

Formula: cost_signal[h] = max(0, base_cost × (1.0 - 0.9 × solar_frac))

Where: solar_frac = solar_W[h] / max(solar_W)

Variables:

base_cost = tariff or peak_tariff ($/kWh) for hour h

solar_frac = Normalized solar output (0.0 = no sun, 1.0 = peak sun)

0.9 = Maximum discount fraction (90% off at peak solar)

Source: ProjectController.php lines 361–364: buildCostSignal()

Worked Example:

tariff = $0.15/kWh, solar at hour noon = 0.95 of max

cost = max(0, 0.15 × (1.0 - 0.9 × 0.95)) = 0.15 × (1.0 - 0.855) = 0.15 × 0.145 = $0.0218/kWh

### 12.26 Window Cost Sum (Scheduler)

Formula: cost_window(start, runHours) = sum_{h=start}^{start+runHours-1} cost_signal[h]

Variables:

start = Window start hour (integer, 0–23)

runHours = Required run duration (integer hours)

cost_signal[h] = Marginal cost at hour h ($/kWh)

Source: ProjectController.php lines 400–408: exhaustive window search in pickBestIntervals()

### 12.27 Annual Energy ×365

Formula: E_annual = E_daily × 365

For each source (solar, grid, generator, battery):

E_daily = sum_{h=0}^{23} P[h] / 1000  (kWh, summing hourly watts)

Variables:

E_annual = Annual energy (kWh/year)

E_daily = Simulated daily energy from representative Monday in the selected month

Source: FinancialAnalysisService.php lines 75–79

Worked Example:

Daily solar used = 32 kWh → Annual = 32 × 365 = 11,680 kWh/year

### 12.28 Weighted Average Tariff

Formula: weighted_tariff = (off_peak_hours × tariff + peak_hours × peak_tariff) / 24

Variables:

off_peak_hours = 24 - (peak_hours_end - peak_hours_start)

peak_hours = peak_hours_end - peak_hours_start

tariff = off-peak rate ($/kWh)

peak_tariff = peak rate ($/kWh)

Source: FinancialAnalysisService.php lines 89–95

Worked Example:

tariff = $0.10, peak_tariff = $0.22, peak hours = 8 (08:00–16:00)

off_peak = 16h, peak = 8h

weighted = (16 × 0.10 + 8 × 0.22) / 24 = (1.60 + 1.76) / 24 = $0.140/kWh

### 12.29 Annual Savings Formula

Formula: annual_savings = total_cost_without - (grid_cost + gen_cost + maintenance_cost)

Where:

total_cost_without = baseline_grid_cost + baseline_gen_cost (no solar/battery simulation)

grid_cost = grid_kwh_annual × weighted_tariff

gen_cost = sum_{h} F(P_gen[h]) × fuel_price × 365 (affine model, hourly)

maintenance_cost = sum of solar_system.annual_maintenance_cost

Standard: Economic dispatch comparison methodology

Source: FinancialAnalysisService.php lines 156–159

### 12.30 Simple Payback

Formula: payback_years = total_investment / annual_savings

Variables:

total_investment = solar_installation_cost + battery_purchase_cost ($)

annual_savings = as above ($)

Source: FinancialAnalysisService.php lines 167–169

Worked Example:

Investment = $45,000, Annual savings = $6,200 → Payback = 45,000 / 6,200 = 7.3 years

### 12.31 LCOE (Levelized Cost of Energy)

Formula: LCOE = (C_install + C_maintenance × N) / (E_solar_annual × N)

Variables:

LCOE = Levelized Cost of Energy ($/kWh)

C_install = Solar installation cost ($)

C_maintenance = Annual maintenance cost ($/year)

N = Projection horizon = 25 years

E_solar_annual = Annual solar generation (kWh)

Standard: NREL / IEA definition of LCOE

Source: FinancialAnalysisService.php lines 173–177

Worked Example:

Install = $30,000, Maintenance = $500/yr, Solar = 15,000 kWh/yr, N=25

LCOE = (30,000 + 500×25) / (15,000×25) = 42,500 / 375,000 = $0.1133/kWh

### 12.32 Panel Degradation Factor

Formula: degrad_factor(year) = (1 - 0.005)^year = 0.995^year

Variables:

degrad_factor = Remaining output fraction after year years

0.005 = 0.5% annual degradation rate (crystalline silicon industry standard)

year = Years since installation (1–25)

Standard: IEC 61215 (PV module qualification); typical crystalline silicon degradation

Source: FinancialAnalysisService.php line 23: PANEL_DEGRADATION = 0.005; line 202

Worked Example:

Year 10: factor = 0.995^10 = 0.9511 → 4.89% total degradation

Year 25: factor = 0.995^25 = 0.8822 → 11.78% total degradation

### 12.33 Battery Replacement Year

Formula: years_to_eol = max(0, rated_cycle_life / 365 - age_years)

replacement_year = ceil(years_to_eol)

Variables:

rated_cycle_life = Chemistry-rated cycle count (e.g. 4,000 for LFP)

365 = Assumed 1 full cycle per day

age_years = Battery age from installation_date to today

Source: FinancialAnalysisService.php lines 185–191

Worked Example:

LFP battery, 4,000 cycles, age = 2.5 years

years_to_eol = max(0, 4000/365 - 2.5) = max(0, 10.96 - 2.5) = 8.46 → replacement_year = 9

### 12.34 Cumulative 25-Year Net Benefit

Formula (per year y):

year_net(y) = annual_savings × (0.995^y) - battery_replacement_cost(y)

cumulative(y) = -total_investment + sum_{i=1}^{y} year_net(i)

payback_year = first y where cumulative(y) > 0

Source: FinancialAnalysisService.php lines 199–209

### 12.35 Solar Self-Consumption

Formula: self_consumption% = (solar_used_kwh + battery_charged_solar_kwh) / solar_generated_kwh × 100

Variables:

solar_used_kwh = Solar energy consumed directly by load (kWh)

battery_charged_solar_kwh = Solar energy stored in batteries (kWh)

solar_generated_kwh = Total solar generation (kWh)

Source: SourceDispatchService.php lines 379–381

Worked Example:

Solar generated = 30 kWh, directly used = 20 kWh, stored = 7 kWh

Self-consumption = (20 + 7) / 30 × 100 = 90%

### 12.36 Backup Hours Formulas

Runtime at max discharge:

T_max = C_usable_kWh / P_max_discharge_kW

Runtime at average discharge:

T_avg = C_usable_kWh / (P_max_discharge_kW / 2)

Current runtime (from current SOC):

T_current = C_available_kWh / (P_max_discharge_kW / 2)

Variables:

C_usable_kWh = Usable capacity = nominal × DoD × age_factor

C_available_kWh = C_usable_kWh × current_SOC

P_max_discharge_kW = nominal_capacity_kWh × C_rate_discharge

Source: Battery.php lines 110–127

Worked Example:

Nominal = 20 kWh, DoD = 0.90, age_factor = 0.95 → C_usable = 17.1 kWh

C_rate_discharge = 1C → P_max = 20 × 1.0 = 20 kW

T_max = 17.1 / 20 = 0.855 h; T_avg = 17.1 / 10 = 1.71 h

## 13. ALL ALGORITHMS

### 13.1 Greedy First-Fit Decreasing (Phase Balance)

Name: Greedy FFD Phase Balancer

Purpose: Assign single-phase loads (rooms, floor blocks, socket groups) to three phases (A, B, C) to minimize VA imbalance.

Step-by-step pseudocode:

1. Collect all 1-phase load BLOCKS (room, floor-own, socket panel) each with VA total

2. Sort blocks descending by VA (largest first — FFD heuristic)

3. Initialize phase_va = {A: 0, B: 0, C: 0}

4. For each block in sorted order:

a. ph = argmin(phase_va)  -- pick the phase with least VA

b. Assign block to ph

c. phase_va[ph] += block.va

5. Record optimal_phase per block

6. Write back optimal_phase to room/floor entities in DB

Time Complexity: O(n log n) for sort + O(n) for greedy assignment = O(n log n)

Why chosen: FFD is optimal for bin-packing in practice; simple; produces ≤ 11/9 OPT + 6/9 waste bound.

Source: PhaseBalanceController.php lines 217–233 (buildingReport())

### 13.2 Exhaustive Window Search (No-Split Scheduling)

Name: Cheapest Consecutive Window

Purpose: Find the single cheapest contiguous block of runHours within the allowed window [winStart, winEnd] of the cost signal.

Step-by-step pseudocode:

1. best_cost = INF; best_start = winStart

2. For s = winStart to (winEnd - runHours):

a. cost = sum(costSignal[s .. s+runHours-1])

b. If cost < best_cost:

best_cost = cost; best_start = s

3. Return interval [{start: best_start, end: best_start + runHours}]

Time Complexity: O(W × H) where W = window width, H = runHours; effectively O(24×24) = O(576) constant

Why chosen: Guarantees global optimum for contiguous scheduling; window is bounded to 24 hours so exhaustive is fast.

Source: ProjectController.php lines 399–408: pickBestIntervals() maxSplits=0 branch

### 13.3 Greedy Cheapest Hours (Split Scheduling)

Name: Cheapest Individual Hours Picker

Purpose: When splits are allowed (max_interruptions > 0), pick the runHours cheapest individual hours from the window, then group consecutive hours into intervals.

Step-by-step pseudocode:

1. Build hour_costs = {h: costSignal[h] for h in [winStart, winEnd)}

2. Sort hour_costs ascending by cost

3. chosen = first runHours hours from sorted list

4. Sort chosen ascending (re-order by hour number)

5. Group consecutive hours into intervals:

intervals = []

group_start = chosen[0]

for i in 1..len(chosen):

if chosen[i] != chosen[i-1] + 1:

intervals.append({start: group_start, end: chosen[i-1]+1})

group_start = chosen[i]

intervals.append({start: group_start, end: last+1})

6. Return intervals

Time Complexity: O(W log W) for sort + O(H) for grouping

Why chosen: Optimal when splits are allowed (selects globally cheapest individual hours); grouping minimizes number of interval records.

Source: ProjectController.php lines 411–433: pickBestIntervals() maxSplits>0 branch

### 13.4 Improvement Guard

Name: Cost Improvement Gate

Purpose: Before saving a new schedule, verify the new intervals actually improve on the current schedule. Prevents unnecessary DB writes and regressions.

Step-by-step pseudocode:

1. current_best_cost = INF

2. For each existing interval [ivS, ivE] in component.usage_time_intervals:

a. For s2 = ivS to (ivE - runHours):

cost = sum(costSignal[s2 .. s2+runHours-1])

current_best_cost = min(current_best_cost, cost)

3. new_cost = sum(costSignal[h] for h in new_intervals)

4. If new_cost >= current_best_cost - 0.0001:

SKIP (no improvement)

5. Else:

savings = current_best_cost - new_cost

SAVE new intervals; record savings

Time Complexity: O(K × W × H) where K = number of existing intervals (typically ≤ 5)

Why chosen: Idempotent operation; prevents infinite re-optimization loops; preserves user overrides.

Source: ProjectController.php lines 271–295

### 13.5 Seven-Step Dispatch (SourceDispatchService)

Name: Greedy Per-Hour Optimal Dispatch

Purpose: For each of 24 hours, allocate load across solar, batteries, utility, and generator in cost-priority order.

Step-by-step pseudocode (per hour h):

Step 1: PAIRED SOLAR CHARGING

For each solar system (by capacity ratio):

Allocate system's proportional solar output to its paired battery banks

Battery absorbs up to its C-rate; surplus returns to shared pool

Step 2: SOLAR COVERS LOAD

solar_used[h] = min(shared_solar, demand)

surplus = shared_solar - solar_used[h]

remaining = demand - solar_used[h]

Step 3: SURPLUS SOLAR CHARGES UNPAIRED BANKS

Distribute surplus proportionally among unpaired batteries (by headroom)

Step 4: BATTERY DISCHARGE

max_discharge = min(sum of C-rate limits, sum of current_SOC × 1000)

discharge = min(remaining, max_discharge)

Distribute proportionally by current SOC across all banks

Update SOC; guard against SOC < 0

Step 5: UTILITY

utility_used[h] = min(utilityCapW, remaining)

Step 6: GENERATOR

gen_used[h] = min(genCapW, remaining)

Step 7: OPPORTUNISTIC GEN CHARGING

If generator already running AND spare capacity below 85% loading:

Charge batteries with spare AC capacity (apply INV_EFF = 0.95)

Generator AC draw increases by charging load

Time Complexity: O(24 × B) where B = number of battery banks; O(24 × B × S) where S = solar systems

Why chosen: Greedy per-hour is computationally efficient and near-optimal for deterministic day-ahead profiles; full MPC would require forecast uncertainty handling beyond current scope.

Source: SourceDispatchService.php lines 185–346

### 13.6 Paired Battery Charging (Solar System)

Name: Proportional Paired Bank Charger

Purpose: Distribute a solar system's output proportionally among its paired batteries based on available headroom (unfilled capacity).

Step-by-step pseudocode:

1. total_head = sum of (usable - current) for all batteries in bank

2. If total_head == 0: return (all full)

3. For each battery b in bank:

a. head_b = usable_b - current_b

b. share = system_output_W × (head_b / total_head)

c. actual = min(share, C_rate_charge_b × 1000)

d. current_b += (actual / 1000) × one_way_eff_b

e. total_charged += actual

4. Surplus = system_output - total_charged (returned to shared pool)

Source: SourceDispatchService.php lines 193–217

### 13.7 Pool Battery Charging (Unpaired/Surplus Solar)

Name: Pool Charger (Shared Surplus)

Purpose: Distribute shared solar surplus to unpaired battery banks proportionally by headroom.

Algorithm: Same as 13.6 but operates on unpairedIds list and uses shared surplus as input.

Source: SourceDispatchService.php lines 227–248

### 13.8 Opportunistic Generator Charging

Name: Spare-Capacity Gen Charger

Purpose: When generator is already running for load, use spare capacity below 85% loading to charge batteries, improving generator fuel efficiency.

Step-by-step pseudocode:

1. gen_max_for_charging = genCapW × 0.85

2. spare_gen_W = max(0, gen_max_for_charging - gen_used[h])

3. If spare_gen_W == 0: skip

4. total_head = sum of (usable - current) × 1000 for all batteries

5. max_gen_charge_ac = min(spare_gen_W, total_C_rate × 1000, total_head / INV_EFF)

6. For each battery b:

a. ac_W = max_gen_charge_ac × (head_b / total_head)

b. ac_W = min(ac_W, C_rate_b × 1000)

c. current_b += (ac_W × INV_EFF / 1000) × one_way_eff_b

7. gen_used[h] += total_ac_drawn  (generator burns extra fuel for charging)

Note: Generator does NOT start solely to charge (only if already running). INV_EFF = 0.95 (AC-to-DC conversion).

Source: SourceDispatchService.php lines 299–338

### 13.9 PSH Latitude Interpolation

Name: Bilinear PSH Lookup

Purpose: Interpolate Peak Sun Hours for any latitude from a 7-row × 12-column table at 10° latitude increments.

Algorithm: Linear interpolation between bounding latitude bands, with southern-hemisphere season flip (+6 months).

Source: SolarIrradianceService.php lines 169–189. See Section 12.18 for full formula.

### 13.10 Southern Hemisphere Season Flip

Name: Month Index Flip

Purpose: Correct PSH lookup for southern hemisphere by shifting months by 6 (January in SH = July solar conditions).

Algorithm: mi = (mi + 6) % 12 applied when lat < 0.

Source: SolarIrradianceService.php line 175

### 13.11 Group-Max Dedup

Name: Group-Max Component Deduplicator

Purpose: When multiple components share the same group_name at the same entity level, keep only the one with the highest VA. This models mutually exclusive loads (e.g. "backup AC unit" — only one runs at a time).

Step-by-step pseudocode:

1. groups = {}

2. For each component c:

If c.group_name is NULL:

ungrouped.append(c)

Else:

key = entity_id + '|' + group_name

If key not in groups OR c.VA > groups[key].VA:

groups[key] = c

3. active = ungrouped + values(groups)

Source: ScheduleController.php lines 278–289, PhaseBalanceController.php lines 327–347, FinancialAnalysisService.php lines 328–340

### 13.12 Hour Membership Test (Midpoint Sampling)

Name: Hour Active Check

Purpose: Determine whether a component's usage interval covers a given clock hour, using midpoint sampling to handle boundary conditions correctly.

Algorithm:

mid = h + 0.5  (midpoint of hour h)

active = (mid >= start AND mid < end)

OR (end > 24 AND (mid+24) >= start AND (mid+24) < end)  -- overnight wrap

Source: ScheduleController.php lines 313–318, FinancialAnalysisService.php lines 358–365

### 13.13 Battery Depletion Tracking

Name: First-Depletion Sentinel

Purpose: Record the first hour any battery bank reaches zero SOC during discharge.

Algorithm:

battDepletedAt = null

For each hour h:

After discharge step:

For each battery b:

If b.current < 0:

b.current = 0

If battDepletedAt is null:

battDepletedAt = h

Source: SourceDispatchService.php lines 274–279

### 13.14 Phase Current Phasor Sum (Per-Phase)

Name: Complex Phasor Aggregator

Purpose: Compute the total current phasor for all loads on a single phase, accounting for individual power factors.

Algorithm: See Section 12.12 (Phase Current Phasor formula). Sum real and imaginary components across all loads on the phase.

Source: PhaseBalanceController.php lines 294–311

### 13.15 Cost Signal Classification

Name: Hour Classifier

Purpose: Classify each hour as free (solar surplus), cheap (battery), expensive (peak/generator/unmet), or normal (base tariff).

Algorithm:

baseline = tariff ?? gen_marginal_cost ?? 999.0

For each hour h:

If cost_signal[h] == 0:      → free_hours

Elif cost_signal[h] < baseline: → cheap_hours

Elif cost_signal[h] > baseline: → expensive_hours

Else: (standard tariff hour, unlisted)

Source: CostSignalService.php lines 131–152

## 14. CONSTANTS & CONFIGURATION VALUES

All hardcoded constants from every file.

| Constant | Value | File | Meaning | Standard |
| --- | --- | --- | --- | --- |
| PANEL_DEGRADATION | 0.005 (0.5%/yr) | FinancialAnalysisService.php | Annual PV panel output degradation | IEC 61215 crystalline Si |
| PROJECTION_YEARS | 25 | FinancialAnalysisService.php | Financial analysis horizon (years) | Industry standard |
| DF_PROJECT | 0.7 | FinancialAnalysisService.php, ScheduleController.php, ValidationController.php | Project-level diversity factor | IEC 60364-8-1 |
| DEFAULT_WORK_DAYS | Mon–Fri | FinancialAnalysisService.php, ScheduleController.php | Default project work days | — |
| ROOF_COVERAGE_RATIO | 0.17 | SolarIrradianceService.php | Usable roof fraction for PV | IEC sizing practice |
| STC_IRRADIANCE_W | 1000 W/m² | SolarIrradianceService.php | Standard Test Conditions irradiance | IEC 60904-3 |
| CAPACITY_ESTIMATE_PR | 0.75 | SolarIrradianceService.php | Conservative PR for capacity sizing | — |
| PERFORMANCE_RATIO | 0.80 | SolarIrradianceService.php | Operational PR for hourly output | IEC 61724 |
| NASA_TIMEOUT_SEC | 10 s | SolarIrradianceService.php | HTTP timeout for NASA API calls | — |
| NASA_NULL_VALUE | -999 | SolarIrradianceService.php | NASA POWER missing-data sentinel | NASA POWER API spec |
| CACHE_DAYS | 30 days | SolarIrradianceService.php | How long NASA GHI data is cached | — |
| REPRESENTATIVE_DAY | 15 | SolarIrradianceService.php | Day-of-month used as solar representative | Statistical midpoint |
| GEN_OPTIMAL_MAX_LOAD | 0.85 (85%) | SourceDispatchService.php | Max generator loading for opportunistic charging | ISO 8528 optimal band |
| INV_EFF | 0.95 | SourceDispatchService.php | Inverter/rectifier one-way efficiency (AC→DC) | Industry standard |
| BATTERY_DEGRADATION_COST | 0.01 $/kWh | CostSignalService.php | Near-zero cost assigned to battery hours | Economic proxy |
| UNMET_COST | 999.0 $/kWh | CostSignalService.php | Load-shedding sentinel cost | — |
| VOLT | 230 V | PhaseBalanceController.php | Single-phase nominal voltage | IEC 60038 |
| WARN_PCT | 10% | PhaseBalanceController.php | Phase imbalance warning threshold | IEC 60034-26 |
| CRIT_PCT | 20% | PhaseBalanceController.php | Phase imbalance critical threshold | IEC 60034-26 |
| SOCKET_ASSUMED_PF | 0.95 | PhaseBalanceController.php | PF assumed for socket outlets | BS 7671 practice |
| PHASE_ANGLE_A | 0.0 rad | PhaseBalanceController.php | Phase A voltage angle | IEC positive-sequence ABC |
| PHASE_ANGLE_B | 2π/3 rad (120°) | PhaseBalanceController.php | Phase B voltage angle | IEC positive-sequence ABC |
| PHASE_ANGLE_C | 4π/3 rad (240°) | PhaseBalanceController.php | Phase C voltage angle | IEC positive-sequence ABC |
| TOLERANCE_PCT | 0.1% | ValidationController.php | Pass/fail tolerance for validation comparison | — |
| TARGET_PF | 0.95 | ValidationController.php, ValidationReferenceService.php | PF correction target | PENRA requirement |
| CAP_STEP_KVAR | 0.5 kVAR | ValidationController.php | Standard capacitor bank step size | IEC 60831 |
| VOLTAGE_LL | 400 V | ValidationController.php | Line-to-line voltage (3-phase) | IEC 60038 |
| FREQUENCY | 50 Hz | ValidationController.php | System frequency | IEC/European standard |
| OUTLET_VA | 200 VA | SocketDemandService.php | Standard outlet rating | BS 7671 / IEC |
| CF threshold 1 | 50 kVA | SocketDemandService.php | Small building CF boundary (CF=1.00) | CIBSE Guide C |
| CF threshold 2 | 250 kVA | SocketDemandService.php | Medium building CF boundary (CF=0.92) | CIBSE Guide C |
| CF large | 0.85 | SocketDemandService.php | Large building coincidence factor (>250 kVA) | CIBSE Guide C |
| DF_ROOM_TO_FLOOR (office) | 0.85 | DiversityFactorService.php | Office room-to-floor diversity | IEC 60364-8-1 |
| DF_FLOOR_TO_BUILDING (office) | 0.80 | DiversityFactorService.php | Office floor-to-building diversity | IEC 60364-8-1 |
| DEFAULT_BUILDING_DFS room_to_floor | 0.90 | DiversityFactorService.php | Generic IEC default | IEC 60364-8-1 |
| DEFAULT_BUILDING_DFS floor_to_building | 0.80 | DiversityFactorService.php | Generic IEC default | IEC 60364-8-1 |
| DEFAULT_ROOM_DF | 0.80 | DiversityFactorService.php | Generic IEC room coincidence | IEC 60364-8-1 |
| PHASE_IMBALANCE threshold healthy | ≥90% age_factor | Battery.php | Battery health "good" threshold | — |
| PHASE_IMBALANCE threshold fair | ≥80% age_factor | Battery.php | Battery health "fair" threshold | — |
| PHASE_IMBALANCE threshold degraded | ≥70% age_factor | Battery.php | Battery health "degraded"; ≤70% = replace | IEC 62619 |
| no_load_fuel default | 0.30 × F_rated | GeneratorLine.php | Default no-load fuel if not configured | ISO 8528-10 |
| SVG_W | 900 px | SingleLineDiagramPage.jsx | SVG canvas width | — |
| SRC_Y | 60 px | SingleLineDiagramPage.jsx | Y-position of source nodes | — |
| BUS_Y | 200 px | SingleLineDiagramPage.jsx | Y-position of main busbar | — |
| BLD_Y | 340 px | SingleLineDiagramPage.jsx | Y-position of building panels | — |
| NODE_R | 32 px | SingleLineDiagramPage.jsx | Source circle radius | — |
| BUS_H | 6 px | SingleLineDiagramPage.jsx | Busbar height | — |

## 15. SECURITY & AUTHORIZATION

### 15.1 Google OAuth 2.0 Flow (Full Step by Step)

The flow is implemented in GoogleController.php:

Step 1 — Frontend initiates login:

Frontend calls GET /auth/google/redirect?origin=http://localhost:5173. The origin parameter tells the backend where to redirect after authentication.

Step 2 — Session stores origin:

session(['auth_origin' => $request->input('origin')]);

Step 3 — Redirect to Google:

Socialite::driver('google')->redirect();

Laravel Socialite constructs the Google OAuth2 authorization URL with client_id, redirect_uri, scope=openid email profile, and response_type=code. The user is redirected to Google's consent screen.

Step 4 — Google redirects back:

Google redirects to GET /auth/google/callback?code=AUTH_CODE&state=.... This is the registered redirect URI.

Step 5 — Exchange code for tokens:

$googleUser = Socialite::driver('google')->user();

Socialite internally POSTs to https://oauth2.googleapis.com/token with the auth code, receiving access_token and refresh_token.

Step 6 — Upsert user record:

$user = User::updateOrCreate(

['email' => $googleUser->getEmail()],

['name' => $googleUser->getName(), 'google_id' => $googleUser->getId(),

'avatar' => $googleUser->getAvatar(), 'google_token' => $googleUser->token,

'google_refresh_token' => $googleUser->refreshToken,

'email_verified_at' => now(), 'password' => null]

);

If the email exists: updates name, tokens, avatar.

If new: creates a new User row.

password is set to NULL (Google-only accounts have no password).

Step 7 — Issue Sanctum token:

$token = $user->createToken('google-auth')->plainTextToken;

Laravel Sanctum creates a personal_access_tokens record linked to the user. The plain-text token is returned once (never stored, hashed in DB).

Step 8 — Redirect to frontend with token:

return redirect("{$frontendUrl}/auth/callback?token={$token}");

Frontend at /auth/callback reads the token from the URL, stores it in localStorage, then navigates to /dashboard.

Source: GoogleController.php, AuthCallbackPage.jsx

### 15.2 Admin Login Flow

Step 1: POST /api/admin/login with { email, password } (no auth middleware — public route).

Step 2: AdminController::login() finds user by email, verifies password with Hash::check().

Step 3: Checks $user->is_admin === true. If not admin: returns 403.

Step 4: Issues Sanctum token: $user->createToken('admin-login')->plainTextToken.

Step 5: Returns { token, user }. Frontend stores token and navigates to /admin/dashboard.

Source: AdminController.php, AdminLoginPage.jsx

### 15.3 Sanctum Token Issuance

All protected API routes use middleware('auth:sanctum'). The client must send:

Authorization: Bearer {token}

Sanctum hashes the token and looks up the personal_access_tokens table. The user is resolved and attached to $request->user(). Tokens do not expire by default (no expiration set in sanctum.php).

### 15.4 ProjectPolicy Rules

File: ProjectPolicy.php

| Method | Condition | Used For |
| --- | --- | --- |
| view | userRole(userId) !== null | Any access (owner, admin, main, or normal member) |
| update | userRole(userId) in ['admin', 'main'] | Write operations: add components, configure sources |
| delete | project->user_id === user->id | Only the project owner (admin) can delete |

Usage: $this->authorize('view', $project) in controllers enforces these rules automatically.

### 15.5 userRole() Method

File: Project.php lines 66–75

public function userRole(int $userId): ?string

{

if ((int) $this->user_id === $userId) return 'admin';

$member = $this->projectUsers()->where('user_id', $userId)->first();

return $member?->role;

}

Returns: 'admin' (owner), 'main' (editor member), 'normal' (read-only member), or null (no access).

### 15.6 Role Hierarchy

| Role | Who | Can View | Can Edit | Can Delete Project | Can Manage Members |
| --- | --- | --- | --- | --- | --- |
| admin | Project owner (user_id matches) | Yes | Yes | Yes | Yes |
| main | Named member with role='main' | Yes | Yes | No | No |
| normal (viewer) | Named member with role='normal' | Yes | No | No | No |
| (none) | Not owner, not a member | No | No | No | No |

Most endpoints check: in_array($project->userRole($userId), ['admin', 'main']) for write operations.

### 15.7 AdminMiddleware

File: AdminMiddleware.php

if (! $request->user() || ! $request->user()->is_admin) {

return response()->json(['message' => 'Unauthorized. Admin access required.'], 403);

}

Applied to the admin middleware group: GET /api/admin/users, PUT /api/admin/users/{user}, DELETE /api/admin/users/{user}.

### 15.8 SecurityHeaders Middleware

File: SecurityHeaders.php

Every API response has these headers added:

| Header | Value | Purpose |
| --- | --- | --- |
| X-Content-Type-Options | nosniff | Prevent MIME-type sniffing attacks |
| X-Frame-Options | DENY | Prevent clickjacking via iframe embedding |
| X-XSS-Protection | 1; mode=block | Legacy browser XSS filter |
| Referrer-Policy | strict-origin-when-cross-origin | Limit referrer leakage on cross-origin requests |
| Permissions-Policy | camera=(), microphone=(), geolocation=() | Deny access to sensitive browser APIs |

Source: SecurityHeaders.php lines 15–19

### 15.9 FormRequest Validation Protection

All store/update endpoints use Laravel FormRequest classes (StoreProjectRequest, UpdateProjectRequest, etc.) with typed rules. Examples from route definitions:

'month' => 'required|integer|min:1|max:12' — prevents invalid month values

'components.*.model_type' => 'required|in:project,building,floor,room' — enum validation

'phase' => 'nullable|in:A,B,C' — strict enum for phase assignment

### 15.10 SQL Injection Prevention

Approach: All database queries use the Eloquent ORM or Laravel Query Builder with parameterized bindings. The comment in api.php line 47 explicitly states: "SQL injection: Prevented by Eloquent ORM — no raw queries in this project." The single use of selectRaw() in SocketDemandService.php only contains fixed column names with no user input.

## 16. BACKUP SYSTEM

### 16.1 ServerBackupController — Endpoints

File: ServerBackupController.php

GET `/api/projects/{project}/server-backups` — index()

Auth: admin or main role required

Query params: entity_type (optional filter), entity_id (optional filter)

Returns: { data: [{id, entity_type, entity_id, entity_name, created_at}] } — metadata only, no payload

GET `/api/server-backups/{backup}/data` — show()

Auth: admin or main role (via backup→project relationship)

Returns: { data: <full backup JSON> } — complete backup payload decoded from DB

DELETE `/api/server-backups/{backup}` — destroy()

Auth: admin or main role required

Deletes the server_backups record; returns { message: 'Server backup deleted.' }

### 16.2 AutoBackupCommand

No dedicated AutoBackupCommand class was found in the codebase. The Project model has auto_backup_interval and last_auto_backup_at fields, suggesting a planned feature. The backup mechanism is user-triggered via saveProjectToServer().

### 16.3 BackupExporter — Format and Contents

File: BackupExporter.php

The exporter serializes the full hierarchy as nested JSON. Format version = "1.0".

Project export structure:

{

"version": "1.0",

"exported_at": "ISO8601 timestamp",

"project": {

"name": "string",

"building_type": "string|null",

"solar_power": "number|null",

"generator_power": "number|null",

"utility_lines": [{name, power, phases}],

"generator_lines": [{name, power, phases}],

"components": [{component_name, power, phases, power_factor, quantity, group_name, priority, needs_socket, usage_season, usage_day_type, usage_time_intervals}],

"sockets": [{phase_type, power, quantity}],

"buildings": [<building objects>],

"solar_systems": [{name, capacity_kw, is_active, notes}],

"batteries": [{name, chemistry, nominal_voltage_v, capacity_ah_per_unit, quantity, series_count, parallel_count, installation_date, depth_of_discharge, round_trip_efficiency, c_rate_charge, c_rate_discharge, rated_cycle_life, current_soc, is_active, notes, solar_system_name}]

}

}

Building export: {name, area, utility_lines, generator_lines, components, sockets, floors: [<floor objects>]}

Floor export: {name, area, utility_lines, generator_lines, components, sockets, rooms: [<room objects>]}

Room export: {name, area, utility_lines, generator_lines, components, sockets}

Battery pairing: solar_system_name stores the paired solar system name (string) so restore can re-link by name match after solar systems are imported first.

### 16.4 ProjectBackupController — JSON Export/Import

File: ProjectBackupController.php

Export endpoints: GET /api/projects/{project}/backup, /api/buildings/{building}/backup, /api/floors/{floor}/backup, /api/rooms/{room}/backup — delegates to BackupExporter, returns JSON directly for browser download.

Import (`restore()`) logic:

Validates { data: array, overwrite: bool? }

Checks for read-only conflict (user has normal role on existing project with same name → 403)

If project with same name exists and overwrite=false → returns { conflict: true } (HTTP 409)

If overwrite=true: deletes existing project first

Wraps everything in DB::transaction()

Import order: project → utility/generator lines → components → solar_systems → batteries (order matters: batteries reference solar_systems by name) → buildings → floors → rooms

Component types resolved/created by name in bulk (one query per level)

On any failure: DB::rollBack(), returns 500

Save-to-server endpoints: POST /{entity}/{id}/save-backup — creates a ServerBackup record in the DB with the full JSON payload.

Duplicate endpoints: POST /projects/{project}/buildings/{building}/duplicate — exports building JSON, appends " (Copy)" to name, re-imports into same project.

### 16.5 BackupChoiceModal — UI Flow

File: BackupChoiceModal.jsx

The modal is triggered when the user clicks a "Backup" button in any entity page. It presents two options:

Option 1 — Download to Computer:

Calls onDownload() (which triggers downloadJson.js to create a Blob URL and click a hidden <a> tag)

Closes modal immediately

Option 2 — Save to Server:

Calls onSaveToServer() (POST to /api/{entity}/{id}/save-backup)

Shows spinner while saving; shows success confirmation on completion

Error message displayed inline on failure

After success, shows "Close" button; modal can also be dismissed by clicking backdrop

## 17. VALIDATION REFERENCE SYSTEM

### 17.1 ValidationController — Every Endpoint

File: ValidationController.php

Single endpoint: GET /api/validation/case-study (auth:sanctum, no admin required)

Response structure:

{

"project_id": integer,

"project_name": "Validation Reference — 2-Floor Office",

"overall_status": "PASS" | "FAIL",

"tolerance_used": "0.1%",

"system_result": { ...production calc results... },

"reference_answer": { ...independent reference results... },

"comparison": [

{

"field": "Total Apparent Power (VA)",

"system_key": "total_va",

"reference_key": "total_va",

"unit": "VA",

"system_value": float,

"reference_value": float,

"difference_percent": float,

"status": "PASS" | "FAIL" | "SKIP"

},

...

]

}

Comparison fields validated:

Total Apparent Power (VA) — tolerance 0.1%

Total Active Power (W) — tolerance 0.1%

Total Reactive Power (kVAR) — tolerance 0.1%

Max Apparent Power (VA) — tolerance 0.1%

System Power Factor — tolerance 0.1%

Socket Demand (VA) — tolerance 0.1%

Socket Connected (VA) — tolerance 0.1%

Component Active Power (W, diversified) — tolerance 0.1%

PF Correction Recommended — boolean exact match

### 17.2 ValidationReferenceService — All Standards Referenced

File: ValidationReferenceService.php

The service is a hand-computed independent implementation. Standards and formulas used:

| Calculation | Standard | Value Used |
| --- | --- | --- |
| Apparent power S = sqrt(P²+Q²) | IEC 60364-8-1 | — |
| Active power P = S × PF | IEC 60364-8-1 | — |
| Reactive power Q = P × tan(arccos(PF)) | IEC 60364-8-1 | — |
| Office room-to-floor DF | IEC 60364-8-1 / CIBSE | 0.85 |
| Office floor-to-building DF | IEC 60364-8-1 / CIBSE | 0.80 |
| Open-office room coincidence | CIBSE Guide C | 0.80 |
| Meeting-room coincidence | CIBSE Guide C | 0.70 |
| Project-level DF | IEC 60364-8-1 | 0.70 |
| Socket outlet VA | IEC / BS 7671 | 200 VA/outlet |
| Socket tiers (100%/75%/40%) | BS 7671 / IEC | 10/10/rest |
| Coincidence factor small (<50 kVA) | CIBSE | 1.00 |
| Coincidence factor medium (50–250 kVA) | CIBSE | 0.92 |
| Coincidence factor large (>250 kVA) | CIBSE | 0.85 |
| PF correction target | PENRA | 0.95 |
| PF correction threshold | PENRA | 0.85 |
| Capacitor bank step | IEC 60831 | 0.5 kVAR |
| Capacitor delta connection | IEC 60831 | C = Q/(2πfV²) × 1e6 |
| System frequency | IEC / European | 50 Hz |
| Line-to-line voltage | IEC 60038 | 400 V |

Case study inputs (2-Floor Office):

Ground Floor, Open Office: 2,000 VA LED lights (PF=1.00), 3,000 VA computers (PF=0.85), 5,000 VA AC (PF=0.90), 20 socket outlets

First Floor, Meeting Room: 800 VA LED lights (PF=1.00), 500 VA projector (PF=0.95), 8 socket outlets

DF chain for open office: 0.80 × 0.85 × 0.80 × 0.70 = 0.3808

DF chain for meeting room: 0.70 × 0.85 × 0.80 × 0.70 = 0.3332

### 17.3 ValidationPage — What It Shows

File: ValidationPage.jsx

The validation page displays:

Overall status banner — green "PASS" or red "FAIL" with the tolerance (0.1%)

System result panel — key values from the production calculation (total VA, W, kVAR, PF, socket demand)

Reference answer panel — independently computed expected values

Comparison table — row per field: Field Name | System Value | Reference Value | Difference % | Status (PASS/FAIL badge)

Breakdown accordion — expands to show per-component contributions with DF values, socket tier breakdown formula

The page calls GET /api/validation/case-study on load. If the validation project is missing (not seeded), it shows the seeder command instruction.

## 18. NAVIGATION & HIERARCHY

### 18.1 NavigationController — Breadcrumb Endpoints

File: NavigationController.php

The NavigationController resolves human-readable URL path segments (project/building/floor/room names) to DB entities and returns breadcrumb data.

GET `/api/nav/{projectName}`

Finds project by name (URL-decoded) owned by or shared with authenticated user (->firstOrFail())

Returns: { project, buildings: [{...withCount('floors')}] }

GET `/api/nav/{projectName}/{buildingName}`

Finds project then building by name within that project

Returns: { project, building, floors: [{...withCount('rooms')}] }

GET `/api/nav/{projectName}/{buildingName}/{floorName}`

Finds project, building, then floor

Returns: { project, building, floor, rooms: [all rooms ordered by created_at desc] }

GET `/api/nav/{projectName}/{buildingName}/{floorName}/{roomName}`

Full four-level path resolution

Returns: { project, building, floor, room, components: [{...with('componentType')}] }

Path resolution: All segments use urldecode() to handle URL-encoded spaces and special characters. Each level uses ->firstOrFail() which throws a ModelNotFoundException (HTTP 404) if not found.

### 18.2 ProjectSidebar — All Nav Links, Active State

File: ProjectSidebar.jsx

Top-level feature links (always visible above the tree):

| Link Label | Route | Active Detection | Color Accent |
| --- | --- | --- | --- |
| Project name | /projects/{projectId} | Not on building/feature page | indigo |
| Phase Balance | /projects/{projectId}/phase-balance | pathname.endsWith('/phase-balance') | violet |
| Load Schedule | /projects/{projectId}/schedule | pathname.endsWith('/schedule') | amber |
| Financial Analysis | /projects/{projectId}/financial | pathname.endsWith('/financial') | emerald |
| Single-Line Diagram | /projects/{projectId}/single-line | pathname.endsWith('/single-line') | cyan |

Hierarchy tree behavior:

Buildings loaded on mount via GET /api/projects/{id}/buildings

Floors loaded lazily on chevron click: GET /api/buildings/{id}/floors

Rooms loaded lazily on chevron click: GET /api/floors/{id}/rooms

Active entity highlighted in indigo based on URL params (buildingId, floorId, roomId)

Auto-expands tree path when navigating from external link (via useEffect on activeBuildingId/activeFloorId)

### 18.3 Full URL Structure Tree

/ → /dashboard (redirect)

/login                                      LoginPage (public)

/auth/callback                              AuthCallbackPage (receives OAuth token)

/admin/login                                AdminLoginPage

/admin/dashboard                            AdminDashboardPage

/dashboard                                  DashboardPage (project list)

/validation                                 ValidationPage

/defense-prep                               DefensePrepPage

/projects/:projectId                        ProjectPage (overview + total power)

/projects/:projectId/schedule               LoadSchedulePage

/projects/:projectId/phase-balance          PhaseBalancePage

/projects/:projectId/financial              FinancialPage

/projects/:projectId/single-line            SingleLineDiagramPage

/projects/:projectId/buildings/:buildingId                      BuildingPage

/projects/:projectId/buildings/:buildingId/floors/:floorId      FloorPage

/projects/:projectId/buildings/:buildingId/floors/:floorId/rooms/:roomId   NewRoomPage

All routes under /projects/:projectId/... are wrapped in ProjectLayout (renders ProjectSidebar + main content). All are protected by ProtectedRoute (redirects to /login if no user).

## 19. LOAD SCHEDULING SYSTEM

### 19.1 Component Scheduling Fields (with Migration Source)

All scheduling fields are on all four component tables: project_components, building_components, floor_components, room_components.

| Field | Type | Meaning |
| --- | --- | --- |
| load_flexibility | enum: fixed, shiftable, curtailable | Whether load can be time-shifted |
| required_run_hours | integer nullable | Hours/day the shiftable component must run |
| earliest_start_hour | integer nullable (0-23) | Earliest allowed start hour |
| latest_end_hour | integer nullable (1-24) | Latest allowed end hour |
| min_continuous_run | integer nullable | Minimum consecutive run hours |
| max_interruptions | integer nullable | Max schedule splits allowed (0 = no splits) |
| usage_time_intervals | JSON array nullable | Actual schedule: [{start:'HH:MM', end:'HH:MM'}] |
| usage_season | enum: all, summer, winter, spring, autumn | Active season filter |
| usage_day_type | enum: all, weekday, weekend | Day-type filter |
| priority | enum: normal, essential, critical | Critical = always-on (DF=1.0, 24 hrs) |

Source: component migrations in database/migrations/

### 19.2 LoadSchedulePage — All Tabs, All UI Flows

File: LoadSchedulePage.jsx

Tab 1 — Load Profile:

Stacked area chart: Max Load (indigo, no diversity) vs Optimized Load (emerald, with diversity factors)

Reactive power Q (amber line, kVAR) overlay

Day selector buttons (Mon–Sun); defaults to current day of week

Month selector dropdown (Jan–Dec)

Stat cards: Peak kW, Daily kWh, Solar kWh, Battery kWh

Tab 2 — Sources:

Solar generation filled area (yellow/amber)

Utility capacity line (blue dashed)

Generator capacity line (orange dashed)

Sunrise/sunset reference lines

Data source badge: NASA POWER (green), Static Lookup (amber), NASA Fallback (orange)

PSH value displayed; peak sun hours for the selected month/location

Tab 3 — Combined Dispatch:

Stacked area: Solar Used (yellow), Battery Discharge (purple), Utility Used (blue), Generator Used (orange)

Total demand as gray line overlay

Battery charge bands: Solar Charging (faint yellow), Generator Charging (faint orange)

Battery SOC trace (violet line, right y-axis 0-100%)

Unmet demand (red area, only visible if load exceeds all sources)

Hover tooltip: all source values + SOC percentage for that hour

Tab 4 — Shiftable Loads:

Lists all load_flexibility = shiftable components project-wide

Columns: Name, Location, Power (W), Required Run Hours, Window [earliest-latest]

Status: green checkmark if already assigned, grey circle if unassigned

24-cell cost signal row per component: color-coded green/yellow/amber/red/crimson

"Optimize All" button: submits all components to POST /api/projects/{id}/optimize-shiftable

After optimization: shows savings (old cost → new cost → savings %) per component

New intervals displayed as time range tags (e.g. "08:00–12:00", "14:00–15:00")

### 19.3 buildCostSignal() — Full Algorithm Both Layers

File: ProjectController.php lines 321–372

Layer 1 — Monetary cost:

Find first utility line with tariff_per_kwh set

Find first generator line with fuel cost > 0 and consumption > 0

base_monetary = tariff ?? gen_cost_per_kwh ?? null

Peak window: if peak_tariff and peak_hours_end > peak_hours_start: hours in [peak_start, peak_end) use peak_tariff

Layer 2 — Solar irradiance discount:

If location_lat is set: fetch normalized 1 kW solar profile for the month

max_solar = max(solar_W) (normalize to 0–1 range)

solar_frac[h] = solar_W[h] / max_solar

Combined signal computation:

If base_monetary is not null:

base_h = peak_tariff if in-peak else tariff ?? gen_cost

signal[h] = max(0, base_h × (1.0 - 0.9 × solar_frac[h]))

// Up to 90% discount at peak solar

Else:

signal[h] = round(1.0 - solar_frac[h], 6)

// Pure irradiance guide: 0 at peak sun, 1 at night

This ensures loads strongly prefer solar hours, and among tariff periods, heavily discounts the sunniest hours.

### 19.4 pickBestIntervals() — Continuous and Split Paths

File: ProjectController.php lines 385–433

Guards: returns null if runHours <= 0, window invalid, or runHours > window_width.

No-split path (max_interruptions == 0):

best_cost = INF; best_start = winStart

For s = winStart to winEnd-runHours:

cost = sum(costSignal[s .. s+runHours-1])

if cost < best_cost: best_cost=cost; best_start=s

return [{start: best_start, end: best_start+runHours}]

Split path (max_interruptions > 0):

hour_costs = {h: costSignal[h] for h in [winStart, winEnd)}

Sort by cost ascending; take first runHours hours

Sort chosen hours ascending

Group consecutive hours into intervals

return intervals

### 19.5 Improvement Guard — Full Logic

File: ProjectController.php lines 268–295

current_best_cost = PHP_FLOAT_MAX

For each existing interval [ivS, ivE]:

For s2 = ivS to ivE-runHours:

c2 = sum(costSignal[s2 .. s2+runHours-1])

current_best_cost = min(current_best_cost, c2)

new_cost = sum(costSignal[h] for all hours h in new_intervals)

If current_best_cost < PHP_FLOAT_MAX AND new_cost >= current_best_cost - 0.0001:

continue  // no improvement; skip

savings = current_best_cost - new_cost

comp.usage_time_intervals = new_intervals

comp.save()

updated[] += {id, savings, savings_percent}

The 0.0001 epsilon prevents false improvements from floating-point rounding. Preserves user-defined schedules that are already optimal.

### 19.6 ScheduleController — Full Response Structure

Endpoint: GET /api/projects/{project}/schedule?month=1-12&day=1-31

{

"month": 7,

"month_name": "July",

"day": 15,

"location": {"lat": 30.0, "lng": 31.0, "name": "Cairo"},

"sunrise_hour": 5.13,

"sunset_hour": 18.87,

"peak_sun_hours": 7.85,

"solar_capacity_w": 63750.0,

"solar_data_source": "nasa_power",

"utility_capacity_va": 50000,

"generator_capacity_va": 30000,

"solar": [0,0,0,0,0,0,1200,3400,5600,...,0],

"solar_systems": [{"id":1,"name":"Roof South","capacity_kw":20.0,"is_active":true}],

"battery_summary": {

"bank_count": 2,

"total_nominal_kwh": 40.0,

"total_usable_kwh": 34.2,

"average_age_years": 1.5,

"chemistries": ["lithium_lfp"]

},

"cost_rates": {

"tariff_per_kwh": 0.12,

"peak_tariff_per_kwh": 0.22,

"peak_hours_start": 14,

"peak_hours_end": 20,

"generator_cost_per_kwh": 0.27,

"generator_rated_kw": 50.0,

"generator_rated_lph": 15.0,

"generator_no_load_lph": 4.5,

"fuel_cost_per_liter": 0.90,

"currency_symbol": "$"

},

"days": {

"monday": {

"load_max": [<24 float watts>],

"load_optimized": [<24 float watts>],

"hourly_kvar": [<24 float kvar>],

"dispatch_max": {

"solar_used": [24 floats],

"battery_charged_solar": [24 floats],

"battery_charged_gen": [24 floats],

"battery_charged": [24 floats],

"battery_discharged": [24 floats],

"battery_remaining_capacity_kw": [24 floats],

"utility_used": [24 floats],

"generator_used": [24 floats],

"unmet": [24 floats],

"unmet_hours": [],

"max_unmet_kw": 0.0,

"battery_depleted_at_hour": null,

"battery_soc_trace": [24 floats 0-1],

"stats": {

"solar_hours": int, "solar_kwh": float,

"solar_generated_kwh": float, "solar_self_consumption": float,

"battery_discharged_kwh": float, "battery_charged_kwh": float,

"battery_efficiency_loss_kwh": float, "final_soc": float,

"utility_hours": int, "utility_kwh": float,

"generator_hours": int, "generator_kwh": float,

"generator_efficiency_avg": float,

"unmet_kwh": float, "total_load_kwh": float

}

},

"dispatch_optimized": { ...same structure... }

},

"tuesday": { ... },

"wednesday": { ... },

"thursday": { ... },

"friday": { ... },

"saturday": { ... },

"sunday": { ... }

}

}

## 20. PHASE BALANCE SYSTEM

### 20.1 PhaseBalancePage — UI Sections

File: PhaseBalancePage.jsx

The page is organized per building:

Section 1 — Actual Distribution:

Shows the current real phase assignment (from saved phase fields on components):

Phase A / B / C: VA load, current (A), percentage of total

Unassigned VA (loads without a phase assignment)

Status badge: Balanced / Warning / Critical

Imbalance % and neutral current (A)

Method note: "phasor_sum_with_pf"

Section 2 — Optimal Distribution (Greedy FFD Result):

Shows what the phase balance would be after applying the greedy algorithm:

Same metrics as actual but for the simulated optimal assignment

"Apply Optimal Assignment" button: POST to /api/buildings/{id}/apply-optimal-phase

Section 3 — Block Assignments Table:

Lists each load block (room, floor-own, socket group) with:

Block type (room / floor_own / socket / building)

Block name and floor

VA amount

Optimal phase assignment (A/B/C)

Section 4 — Floor/Room Breakdown:

Expandable floor accordion showing each room with:

Actual phase (from DB)

Optimal phase (from greedy result)

VA on that phase

"Reassign" button per room: POST to /api/rooms/{id}/assign-phase

### 20.2 PhaseBalanceController — Every Method

File: PhaseBalanceController.php

`project(Request, Project): JsonResponse`

Auth: userRole check (any role)

Loads all buildings with components eagerly

Maps each building through buildingReport()

Returns: { buildings: [<building report objects>] }

`building(Request, Building): JsonResponse`

Auth: userRole check (any role)

Returns single buildingReport() result

`floor(Request, Floor): JsonResponse`

Auth: userRole check

Collects 1-phase loads from floor own components + all room components

Adds socket demand as a synthetic load block

Runs simple greedy (no per-building-type block grouping)

Returns standard simpleResponse() structure

`assignRoom(Request, Room): JsonResponse`

Auth: admin or main role

Validates: phase in {A, B, C, null}

Updates all 1-phase components in the room: components()->where('phases','1phase')->update(['phase' => $phase])

Returns: { message: 'Phase updated.' }

`applyOptimalBuilding(Request, Building): JsonResponse`

Auth: admin or main role

Runs buildingReport() to get block_assignments

For each block assignment: updates component phases in DB by block type (room, floor_own, building)

Socket blocks are virtual (no DB record) — skipped

Returns: { message: 'Optimal phase assignment applied.' }

`buildingReport(Building, SocketDemandService): array` (private)

Core method:

Collects 1-phase load blocks (building own, floor own, socket panels, rooms)

Sorts by VA descending

Runs greedy FFD assignment

Computes phasor currents per phase using computePhaseCurrent()

Computes neutral current using computeNeutralCurrent()

Builds actual distribution from saved phase fields

Returns complete report with actual + optimal sections

`computePhaseCurrent(loads, phaseAngle): array` (private)

Returns { real, imag, magnitude } — full complex phasor for all loads on one phase. See Section 12.12.

`computeNeutralCurrent(phaseLoads): float` (private)

Returns scalar |I_N| from phasor sum of three phase currents. See Section 12.13.

`extract1ph(components): array` (private)

Filters for 1-phase components, deduplicates by group_name (max-VA wins), returns [{name, va, pf, phase}].

`consensusPhase(components): ?string` (private)

Returns 'A'|'B'|'C' if all 1-phase components agree on a phase, 'mixed' if they disagree, null if none assigned.

`imbalanceStatus(va): array` (private)

Computes [status, imbalance_pct] using VA/230 approximation for magnitude comparison.

`extractSimple(components, &loads1ph, &va3ph, &count3ph): void` (private)

Floor-level helper: splits components into 1-phase and 3-phase lists with group-max dedup.

`simpleResponse(entityType, entityId, entityName, loads1ph, va3ph, count3ph): JsonResponse` (private)

Formats the floor-level greedy result with recommendations text.

### 20.3 Complete Phasor Math for Neutral Current

Phase angles (positive-sequence ABC):

Phase A: θ_V = 0 rad

Phase B: θ_V = 2π/3 rad = 120°

Phase C: θ_V = 4π/3 rad = 240°

Per load on phase X:

|I| = VA_load / 230        (current magnitude)

φ   = arccos(PF_load)      (lag angle)

θ_I = θ_V(X) - φ          (current angle lags voltage by φ)

I_real += |I| × cos(θ_I)

I_imag += |I| × sin(θ_I)

Phase total phasor:

I_X = (I_real_X, I_imag_X)

|I_X| = sqrt(I_real_X² + I_imag_X²)

Neutral current (Kirchhoff):

I_N_real = I_A_real + I_B_real + I_C_real

I_N_imag = I_A_imag + I_B_imag + I_C_imag

|I_N| = sqrt(I_N_real² + I_N_imag²)

Perfectly balanced example (all phases 2,300 VA, PF=1.0):

Phase A: I=(10,0); Phase B: I=(-5,+8.66); Phase C: I=(-5,-8.66)

Sum: (0,0) → |I_N| = 0 A

Imbalanced example (A=4,600 VA, B=0 VA, C=0 VA, PF=1.0):

Phase A: I=(20,0); B: I=(0,0); C: I=(0,0)

Sum: (20,0) → |I_N| = 20 A (full phase A current flows back in neutral)

### 20.4 Greedy FFD Per Building

The greedy algorithm processes each building independently:

Collect all load blocks from the building:

Building-level own 1-phase components (one block)

Per floor: floor-own 1-phase components (one block each)

Per floor: socket demand from SocketDemandService::floorResult() (one block each)

Per room: room total 1-phase VA (one block each)

Sort all blocks descending by VA (FFD order)

Initialize optVa = {A:0, B:0, C:0}, phaseLoads = {A:[], B:[], C:[]}

For each block in sorted order:

ph = argmin(optVa) — always assign to the lightest phase

optVa[ph] += block.va

Append all individual loads from this block to phaseLoads[ph]

Compute phasor currents and neutral current for the optimal assignment

This ensures the greedy respects the natural granularity of electrical panels: rooms are assigned as units (you don't split a room's loads across phases), which matches real installation practice.

## 21. FINANCIAL ANALYSIS SYSTEM

### 21.1 FinancialPage — UI Sections

File: FinancialPage.jsx

Month Selector:

Top-right dropdown (Jan–Dec). Changing month re-fetches from /api/projects/{id}/financial-analysis?month=N. Default: current month.

Summary Cards Row:

6 KPI cards: Annual Solar kWh | Grid kWh | Generator kWh | Annual Savings (with % badge) | Simple Payback (years) | LCOE ($/kWh)

Energy Mix Pie Chart:

Recharts PieChart showing Solar % / Grid % / Generator % of total annual load. Labels show percentages.

Cost Comparison Section:

Two columns side by side:

"Without Solar/Battery": grid cost + generator cost

"With Solar/Battery": grid cost + generator cost + maintenance

Savings highlighted in green (or red if negative)

Weighted tariff $/kWh and generator marginal $/kWh displayed

Investment Summary:

Solar installation cost

Battery purchase cost

Total investment

Simple payback years

25-Year Projection Chart:

Recharts LineChart:

X-axis: Year 1–25

Y-axis: Cumulative net cash flow ($)

Line color: red below zero, green above zero (via gradient)

Reference line at y=0 (break-even)

Payback year dot with label (first year cumulative crosses zero)

Hover tooltip: year + cumulative value formatted as currency

Battery Replacement Markers:

Vertical reference lines on the 25-year chart at years where battery replacement cost is due (diamond markers).

Data Note:

Footer shows which month was used, data source (NASA/static), and degradation assumption (0.5%/yr).

### 21.2 FinancialController — Response JSON Structure

File: FinancialController.php

Endpoint: GET /api/projects/{project}/financial-analysis?month=1-12

{

"annual_energy": {

"solar_kwh": 11680.0,

"grid_kwh": 28470.0,

"generator_kwh": 2190.0,

"battery_loss_kwh": 365.0,

"total_load_kwh": 42340.0,

"solar_percent": 27.6,

"grid_percent": 67.2,

"generator_percent": 5.2

},

"annual_costs": {

"grid_cost": 3988.0,

"generator_cost": 1971.0,

"maintenance_cost": 500.0,

"total_with_solar": 6459.0,

"total_without_solar": 9840.0,

"weighted_tariff": 0.1400,

"generator_cost_per_kwh": 0.2700

},

"savings": {

"annual_savings": 3381.0,

"savings_percent": 34.4

},

"investment": {

"solar_installation": 30000.0,

"battery_purchase": 8000.0,

"total_investment": 38000.0

},

"payback": {

"simple_payback_years": 11.2,

"lcoe_solar_per_kwh": 0.1133

},

"projection_25yr": {

"cumulative_net_by_year": [-38000, -35300, -32450, ..., 28700],

"payback_year": 14,

"total_25yr_benefit": 28700.0

},

"currency_symbol": "$"

}

### 21.3 FinancialAnalysisService — Full Step-by-Step

File: FinancialAnalysisService.php

STEP 1: Build load and solar profiles

collectComponents(project): traverses full hierarchy, applies diversity chain, returns component list

buildHourlyW(components, 'monday', month): builds 24-element W array with group-max dedup, season/day filters, diversity factors

Determines solar capacity: existing mode uses named systems sum (or legacy field); max mode uses area-based estimate

Fetches getHourlyOutputWatts() for the month (day=15, PR=0.80)

Applies utility (×0.8) and generator (×0.8) capacity derating

STEP 2: Normal dispatch simulation

Calls SourceDispatchService::dispatch() with load, solar, utility cap, gen cap, active batteries, solar systems

Extracts daily stats: solar_kwh, utility_kwh, generator_kwh, battery_loss_kwh, total_load_kwh

Annualizes by ×365

STEP 3: Weighted average tariff

Finds first utility line with tariff

If peak tariff set: weighted = (off_peak_h × tariff + peak_h × peak_tariff) / 24

Else: weighted = tariff

STEP 4: Generator annual cost

Uses affine fuel model per hour: sum(fuelAtLoadKw(kW) × fuel_price) × 365

More accurate than flat kWh rate (captures part-load inefficiency)

STEP 5: Annual costs with solar

grid_cost = grid_kwh_annual × weighted_tariff

gen_cost = affine_daily × 365

maintenance = sum(solar_system.annual_maintenance_cost)

total_with_solar = grid_cost + gen_cost + maintenance

STEP 6: Baseline dispatch (no solar, no battery)

Re-runs dispatch with zero solar profile and batteries=null

Computes baseline grid cost and gen cost using same affine model

total_without = baseline_grid_cost + baseline_gen_cost

STEP 7: Savings

annual_savings = total_without - total_with_solar

savings_pct = annual_savings / total_without × 100

STEP 8: Investment and payback

total_investment = solar_installation_cost + battery_purchase_cost

simple_payback = total_investment / annual_savings

LCOE = (install + maintenance × 25) / (solar_kwh_annual × 25)

STEP 9: Battery replacement schedule

For each battery: years_to_eol = rated_cycle_life/365 - age_years

replacement_year = ceil(years_to_eol) if within 1–25 year horizon

Groups by year: battReplByYear[year] += replacement_cost

STEP 10: 25-year cumulative projection

running_net = -total_investment (start negative)

For year y = 1 to 25:

degrad_factor = (1 - 0.005)^y

year_net = annual_savings × degrad_factor - battReplByYear[y]

running_net += year_net

cumulative[y] = running_net

If running_net > 0 for first time: payback_year = y

STEP 11: Energy mix percentages

solar_pct = solar_kwh_annual / total_load_kwh × 100

grid_pct = grid_kwh_annual / total_load_kwh × 100

gen_pct = gen_kwh_annual / total_load_kwh × 100

## 22. POWER SOURCES (ALL 4)

### 22.1 Utility Lines

Model: UtilityLine.php

Fillable fields:

| Field | Type | Meaning |
| --- | --- | --- |
| name | string | Display name (e.g. "Main Grid Feed") |
| power | float | Rated capacity (VA) |
| phases | string | '1phase' or '3phase' |
| tariff_per_kwh | float nullable | Off-peak electricity tariff (currency/kWh) |
| peak_tariff_per_kwh | float nullable | Peak-period tariff (currency/kWh) |
| peak_hours_start | integer nullable | Peak period start hour (0–23) |
| peak_hours_end | integer nullable | Peak period end hour (1–24) |

Relationship: polymorphic lineable — can belong to Project, Building, Floor, or Room.

Controller CRUD: UtilityLineController.php

GET /{entity}/{id}/utility-lines — index: lists all lines for the entity

POST /{entity}/{id}/utility-lines — store: creates new line

PUT /utility-lines/{line} — update: validates power>=0, tariff>=0, peak_tariff>=0

DELETE /utility-lines/{line} — destroy

Tariff logic in dispatch: Utility capacity used as hard cap in SourceDispatchService. Priority 5 in dispatch order (after solar, battery). In financial analysis, weighted_tariff is computed from off-peak/peak weighted average.

Derating for dispatch: utilCapW = utility_lines.sum('power') × 0.8 — 20% derating for safety margin.

### 22.2 Generator Lines

Model: GeneratorLine.php

Fillable fields:

| Field | Type | Meaning |
| --- | --- | --- |
| name | string | Display name |
| power | float | Rated capacity (VA) |
| phases | string | '1phase' or '3phase' |
| fuel_cost_per_liter | float nullable | Fuel price (currency/L) |
| fuel_consumption_lph | float nullable | Full-load fuel consumption (L/h) |
| no_load_fuel_lph | float nullable | No-load fuel consumption (L/h); default=0.30×rated |
| min_load_pct | integer nullable | Minimum recommended loading % |
| optimal_load_pct | integer nullable | Optimal loading % for best fuel efficiency |

Computed property `cost_per_kwh`:

getCostPerKwhAttribute(): (fuel_cost × fuel_consumption_lph) / (power / 1000) at rated load.

Four model methods:

getCostPerKwhAttribute(): flat-rate cost at 100% load — backward compatible accessor

fuelAtLoadKw(kW): affine F(P) = F_0 + (F_rated - F_0) × P/P_rated — hourly fuel (L/h) at any kW

costPerKwhAtLoad(kW): fuel cost per kWh at specific load: fuel_price × fuelAtLoadKw(kW) / kW

marginalCostPerKwh(): d(fuel_cost)/d(kW) = fuel_price × (F_rated - F_0) / P_rated — used in cost signal

Relationship: polymorphic generable — can belong to Project, Building, Floor, or Room.

Controller: GeneratorLineController.php — same CRUD pattern as UtilityLineController.

### 22.3 Solar Systems

Model: SolarSystem.php

Fillable fields:

| Field | Type | Meaning |
| --- | --- | --- |
| project_id | integer | Owning project (project-scoped only) |
| name | string | Display name (e.g. "Roof South Array") |
| capacity_kw | float | Nameplate DC capacity (kW) |
| is_active | boolean | Whether included in dispatch |
| notes | text nullable | Free-form notes |
| installation_cost | float nullable | Total installed cost ($) |
| annual_maintenance_cost | float nullable | Annual O&M cost ($) |
| panel_lifetime_years | integer nullable | Expected panel life |

Relationship: hasMany(Battery::class) — batteries can be paired to a solar system.

Controller: SolarSystemController.php

GET /api/projects/{project}/solar-systems — index

POST /api/projects/{project}/solar-systems — store

PUT /api/solar-systems/{solarSystem} — update

DELETE /api/solar-systems/{solarSystem} — destroy

solar_source modes (on Project model):

'existing': uses named solar systems sum (or existing_solar_power legacy field)

'max': uses roof area estimate estimateCapacityW(total_area_m2)

In dispatch: Each solar system's proportional share = capacity_kw / total_capacity_kw. Its paired batteries get first access to its output before the shared pool.

### 22.4 Batteries

Model: Battery.php

Fillable fields:

| Field | Type | Meaning |
| --- | --- | --- |
| project_id | integer | Owning project |
| name | string | Display name |
| chemistry | string | lead_acid_flooded, lead_acid_agm, lead_acid_gel, lithium_lfp, lithium_nmc |
| nominal_voltage_v | float | Cell string voltage (V) |
| capacity_ah_per_unit | float | Amp-hour rating per unit (Ah) |
| quantity | integer | Number of units |
| series_count | integer | Series string count |
| parallel_count | integer | Parallel string count |
| installation_date | date | For age calculation |
| depth_of_discharge | float | DoD fraction (0–1) |
| round_trip_efficiency | float | RTE fraction (0–1) |
| c_rate_charge | float | Charge rate (C) |
| c_rate_discharge | float | Discharge rate (C) |
| rated_cycle_life | integer | Manufacturer rated cycles |
| current_soc | float | Current state of charge (0–1) |
| is_active | boolean | Include in dispatch |
| notes | text nullable | Free-form notes |
| solar_system_id | integer nullable | Paired solar system (FK) |
| purchase_cost | float nullable | Purchase cost ($) |
| replacement_cost | float nullable | Future replacement cost ($) |

Computed accessors (appended, not stored):

| Accessor | Formula | Units |
| --- | --- | --- |
| age_years | installation_date.diffInDays(now) / 365.25 | years |
| nominal_capacity_kwh | nominal_v × ah_per_unit × quantity / 1000 | kWh |
| age_factor | max(0.70, 1 - age_years × degradation_per_year) | dimensionless |
| usable_capacity_kwh | nominal_kwh × DoD × age_factor | kWh |
| current_available_kwh | usable_kwh × current_soc | kWh |
| max_charge_power_kw | nominal_kwh × c_rate_charge | kW |
| max_discharge_power_kw | nominal_kwh × c_rate_discharge | kW |
| runtime_hours_at_max_discharge | usable_kwh / max_discharge_kw | h |
| runtime_hours_at_average_discharge | usable_kwh / (max_discharge_kw / 2) | h |
| current_runtime_hours | current_available_kwh / (max_discharge_kw / 2) | h |
| health_status | age_factor ≥0.90→'good'; ≥0.80→'fair'; ≥0.70→'degraded'; else 'replace' | string |
| remaining_calendar_life_years | max(0, calendar_life_years - age_years) | years |

Chemistry integration: BatteryChemistryService::getDefaults(chemistry) provides degradation_per_year for age_factor and calendar_life_years for remaining life calculation.

Controller: BatteryController.php

GET /api/projects/{project}/batteries — index with all computed properties

POST /api/projects/{project}/batteries — store; chemistry preset auto-fills missing parameters

GET /api/projects/{project}/battery-runtime — aggregated runtime summary

GET /api/batteries/{battery} — show single battery

PUT /api/batteries/{battery} — update

DELETE /api/batteries/{battery} — destroy

POST /api/batteries/{battery}/reset-soc — resets current_soc to depth_of_discharge (full charge)

POST /api/batteries/{battery}/runtime-at-load — calculates runtime at a specific kW load

GET /api/battery-chemistry-defaults — public endpoint returning all chemistry presets

## 23. EXPORT FEATURES

### 23.1 Excel Export — All 3 Sheets, All Columns

File: exportToExcel.js

Uses SheetJS (xlsx library). Called via exportProjectExcel(projectId, projectName, powerData, engineerName).

Sheet 1 — "Summary":

Built by buildSummarySheet(). Array-of-arrays format (no header row styling on this sheet).

Rows:

Title: "Power Profile — Electrical Load Analysis Export"

Project, Engineer, Date, Standards

DEMAND SUMMARY: Max Load (VA + W), Optimized Load (VA + W), Diversity Reduction %

REACTIVE POWER: System PF, PF Status, Q (kVAR), Max Q, Line Current (A)

PRIORITY BREAKDOWN table: Critical / Essential / Normal rows with Max kVA, Max kW, Opt kVA, Opt kW

SOCKETS section (if sockets present): Connected (VA), Estimated Demand (VA)

CAPACITOR BANK section (if correction needed): Required Bank (kVAR), Capacitor Value (μF), Line Current after, Reduction %

Column widths: [30, 18, 14, 14, 14] characters

Sheet 2 — "Components":

Built by buildComponentsSheet(). Styled with applyStyles().

Columns (12 total):

| Column | Content |
| --- | --- |
| Level | Project / Building / Floor / Room |
| Location | Hierarchy path (e.g. "BuildingA > Floor1 > Room2") |
| Component Name | componentType.name |
| Power (W) | Rated power |
| Quantity | Count |
| Power Factor | PF value |
| Total W | Power × Quantity |
| Phases | 1phase / 3phase |
| Priority | normal / essential / critical |
| Scheduling | fixed / shiftable / curtailable |
| Season | all / summer / winter / spring / autumn |
| Day Type | all / weekday / weekend |

Column widths: [10, 22, 28, 10, 9, 12, 10, 8, 10, 12, 10, 10]

Data is collected by traversing the full hierarchy via API calls: project components → each building → each floor → each room.

Sheet 3 — "Financial":

Built by buildFinancialSheet().

Rows:

FINANCIAL ANALYSIS SUMMARY header

Installation Cost, Annual Savings, Simple Payback, LCOE, 25-Year Net Benefit

Solar Capacity (kW), Battery Capacity (kWh)

Blank row separator

Yearly breakdown table with header: Year | Solar kWh | Savings ($) | Battery Replacement ($) | Net ($) | Cumulative ($)

One row per year 1–25

Column widths: [28, 14, 14, 24, 12, 14]

SheetJS cell styling:

headerStyle(bgColor): bold white text, colored background fill, centered, bottom border

cellStyle(even): alternating F9FAFB/FFFFFF row fill, bottom hair border

applyStyles(ws, headers, rowCount): applies header style to row 0, data style to remaining rows

### 23.2 PDF Export — printPowerReport.js Template

File: printPowerReport.js

Opens a new browser window with a complete HTML report and triggers window.print().

Template sections:

Letterhead:

Power Profile logo (blue box with ⚡ icon)

Brand name "Power Profile" + tagline

Right side: Project name, Engineer name, Date, Reference number (PP-XXXXXX from timestamp)

Load Summary section:

Three cards: Max Load (Unoptimized) VA/W, Optimized Demand VA/W, Diversity Reduction %

Power Factor & Reactive Power section:

Color-coded pill: green (≥95%), amber (≥85%), red (<85%)

Table: PF, Q (kVAR), Max Q, Line Current

Progress bar: gradient red→amber→green from 0.0 to 1.0 with labels at 0.85 and 1.0

Capacitor Bank section:

If PF correction needed: Required bank (kVAR), Capacitor value (μF delta 400V), Target PF, line current before/after, reduction %

If not needed: "No capacitor bank required" green note

If capApplied=true: shows corrected values and green "Applied" banner

Load Breakdown by Priority:

Table: Priority (Critical/Essential/Normal) × Max S / Max P / Optimized S / Optimized P

Socket Outlets section (if sockets > 0):

Connected capacity (VA), Estimated Demand (VA)

Certification/Signature block:

Three fields: Prepared By, Date, Reference No.

Footer:

Standards: IEC 60364-8-1 · PENRA · NEC Article 430 · IEC 60831 · BS 7671 · CIBSE Guide C

Print CSS: A4 size, 12mm margins, print-color-adjust: exact for background fills, page-break-inside: avoid on sections.

### 23.3 Single-Line Diagram — SVG Structure, All Components

File: SingleLineDiagramPage.jsx

The diagram is a pure SVG rendered in React. Canvas: 900px wide × variable height.

Layout constants:

SRC_Y = 60px — Y center of source node row

BUS_Y = 200px — Y center of main busbar

BLD_Y = 340px — Y center of building panel row

NODE_R = 32px — Source circle radius

BUS_H = 6px — Busbar height

Color palette:

| Component | Fill | Stroke | Text |
| --- | --- | --- | --- |
| Solar | #fef9c3 (light yellow) | #ca8a04 (amber) | #713f12 |
| Battery/BESS | #ede9fe (light purple) | #7c3aed (violet) | #4c1d95 |
| Utility/Grid | #dbeafe (light blue) | #2563eb (blue) | #1e3a8a |
| Generator | #ffedd5 (light orange) | #ea580c (orange) | #7c2d12 |
| Main Bus | #1e3a8a (dark blue) | #1e3a8a | — |
| Building | #f0fdf4 (light green) | #16a34a (green) | #14532d |

SVG components:

SourceNode({ cx, cy, r, col, title, line1, line2, icon }):

Outer glow circle (r+4, 40% opacity)

Main circle (r, solid)

Icon text (emoji/symbol at cy-6)

Title text (bold, at cy+7)

Two label lines below circle

BuildingNode({ cx, cy, w, h, col, name, line1, line2, active }):

Optional active glow rect (w+8, h+8, 30% opacity)

Main rect (rounded corners)

Name text (uppercase, bold, at cy-10)

Line1 (large bold value)

Line2 (small gray subtitle)

Wire({ x1, y1, x2, y2, dashed }):

SVG line, stroke #6b7280, width 1.5

Dashed when dashed=true (5,3 pattern)

Breaker({ cx, cy }):

Small square (14×14) with diagonal slash — represents circuit breaker symbol

Source positioning: Sources are spread evenly across the 900px canvas width. The main busbar is a full-width filled rectangle at BUS_Y. Vertical wires connect each source to the busbar with a Breaker symbol at midpoint. Building panels hang below the busbar connected by vertical wires.

Data loading: Parallel API calls on mount: buildings, solar-systems, batteries, generator-lines, utility-lines, total-power.

Tooltip: Hover over any node shows a floating tooltip with detailed data (VA, kW, chemistry, etc.).

## 24. EDGE CASES & ERROR HANDLING

### 24.1 All Try/Catch, Guard Clauses, Fallbacks, and NULL Checks

| File + Function | Condition Handled | Fallback | Why |
| --- | --- | --- | --- |
| SolarIrradianceService::fetchNasaHourlyGhi() | HTTP non-200 response from NASA | Sets nasaFailed=true, returns [] | NASA unavailability must not break calculations |
| SolarIrradianceService::fetchNasaHourlyGhi() | Empty data in NASA response | Sets nasaFailed=true, returns [] | NASA sometimes returns empty for recent dates |
| SolarIrradianceService::fetchNasaHourlyGhi() | All-zero GHI for non-polar location | Sets nasaFailed=true, returns [] | Future dates return -999 nulls which become zeros |
| SolarIrradianceService::fetchNasaHourlyGhi() | Any \Throwable (network timeout, etc.) | nasaFailed=true, returns [] | Network unreachability must not crash the app |
| SolarIrradianceService::getHourlyOutputWatts() | panelCapacityKw <= 0 | Returns 24-element zero array | Division by zero protection |
| SolarIrradianceService::hourlyProfile() | capacityW <= 0 | Returns 24-element zero array | — |
| SolarIrradianceService::hourlyProfile() | cosHa > 1.0 (polar night) | Returns 24 zeros; sets polar_night flag | Latitude/winter combination above Arctic Circle |
| SolarIrradianceService::hourlyProfile() | cosHa < -1.0 (midnight sun) | Distributes flat output over 24h | Arctic summer / polar regions |
| SolarIrradianceService::interpolatePsh() | abs(lat) > 60 | Clamps to 60° | Table only covers 0°–60° |
| SourceDispatchService::dispatchOptimized() | Battery current goes below 0 after discharge | Clamps to 0.0, records battDepletedAt | SOC physically cannot be negative |
| SourceDispatchService::dispatchOptimized() | totalCurrentStep4 <= 0 | Skips discharge; maxDischW = 0 | Prevents division-by-zero in proportional share |
| SourceDispatchService::dispatch() | No active batteries / empty collection | Falls through to dispatchBasic() | Batteries are optional |
| GeneratorLine::fuelAtLoadKw() | ratedKw <= 0 or fRated <= 0 | Returns 0.0 | Prevent division-by-zero |
| GeneratorLine::getCostPerKwhAttribute() | Any parameter <= 0 | Returns null | Unconfigured generators must not produce NaN |
| GeneratorLine::marginalCostPerKwh() | Any parameter <= 0 | Returns 0.0 | — |
| Battery::getAgeFactorAttribute() | Degradation formula results < 0.70 | Clamps to 0.70 (70% floor) | Battery is still operational until 70% capacity |
| Battery::getCurrentAvailableKwhAttribute() | current_soc outside [0,1] | Clamps: max(0, min(1, soc)) | Database corruption guard |
| Battery::getRuntimeHoursAtMaxDischargeAttribute() | max_discharge_power_kw <= 0 | Returns 0 | Division-by-zero |
| CostSignalService::compute() | No utility line with tariff | tariff = null | Partial configuration allowed |
| CostSignalService::compute() | No generator with fuel cost | genCostPerKwh = null | Partial configuration allowed |
| CostSignalService::compute() | No sources configured at all | Cost signal falls through to UNMET_COST = 999 | Valid state: project without sources yet |
| FinancialAnalysisService::analyzeProject() | No utility line found | tariff = 0.0, weightedTariff = 0.0 | Financial calc proceeds with zero grid cost |
| FinancialAnalysisService::analyzeProject() | annualSavings <= 0 or totalInvestment <= 0 | simplePayback = null | Payback undefined for zero/negative savings |
| FinancialAnalysisService::analyzeProject() | lcoeDenominator <= 0 | lcoeSolar = 0.0 | No solar generation = no LCOE |
| FinancialAnalysisService::analyzeProject() | totalWithout <= 0 | savingsPct = 0.0 | Avoid division by zero |
| ProjectController::optimizeShiftable() | runHours <= 0 | Adds to errors[], skips component | Unset constraints produce error message |
| ProjectController::optimizeShiftable() | runHours > window_width | Adds to errors[], skips component | Physically impossible schedule |
| ProjectController::pickBestIntervals() | winEnd <= winStart | Returns null | Invalid window configuration |
| ProjectController::buildCostSignal() | No location set | solarW = array_fill(0,24,0.0) | Location is optional; solar discount is skipped |
| ProjectController::buildCostSignal() | max_solar == 0 | Uses 1.0 as normalizer | Night-time only month edge case |
| ProjectBackupController::restore() | Any \Throwable during import | DB::rollBack(), returns 500 | Partial imports must never corrupt data |
| ProjectBackupController::restore() | Missing project name in backup | Returns 422 with message | Corrupted backup file |
| ProjectBackupController::restore() | User has read-only access to same-name project | Returns 403 | Prevent overwrite of shared projects |
| ProjectBackupController::restore() | Existing project + no overwrite flag | Returns 409 conflict | User must explicitly confirm overwrite |
| ValidationController::show() | Validation project not seeded | Returns 404 with seeder command | Development/setup helper |
| PhaseBalanceController::buildingReport() | Building has no 1-phase loads | Returns empty distribution | Valid state; shows all zeros |
| NavigationController::project/building/floor/room() | Entity not found by name | ->firstOrFail() → 404 | URL names are user-defined, may change |
| ScheduleController::project() | No power sources configured | Returns 422 with error:'no_sources' | Prevents nonsensical empty dispatch result |
| ScheduleController::extractRaw() | usage_time_intervals is JSON string | json_decode() conversion | Legacy data format compatibility |
| BatteryChemistryService::getDefaults() | Unknown chemistry string | Returns null | User may store custom chemistry not in presets |
| exportToExcel.js::collectComponents() | Any API call fails during collection | catch: continue (skip level) | Partial export is better than no export |
| exportToExcel.js::exportProjectExcel() | Financial analysis fetch fails | Sheet 3 is skipped | Financial data is optional |
| printPowerReport.js | window.open() blocked by browser | if (!win) return | Popup blocker protection |
| SolarIrradianceService | NASA returns -999 null values | max(0.0, value) — clamps -999 to 0 | NASA API sentinel value must not produce negative generation |

## 25. QUICK REFERENCE TABLES

### Table 1: All API Endpoints

| Method | Path | Controller@method | Auth | Description |
| --- | --- | --- | --- | --- |
| GET | /api/user | UserController@show | sanctum | Get current user |
| POST | /api/logout | UserController@logout | sanctum | Revoke current token |
| GET | /api/projects | ProjectController@index | sanctum | List user's projects |
| POST | /api/projects | ProjectController@store | sanctum | Create project |
| GET | /api/projects/{project} | ProjectController@show | sanctum | Get project |
| PUT | /api/projects/{project} | ProjectController@update | sanctum+admin/main | Update project |
| DELETE | /api/projects/{project} | ProjectController@destroy | sanctum+owner | Delete project |
| GET | /api/projects/{project}/all-floors | ProjectController@allFloors | sanctum | All floors in project |
| GET | /api/projects/{project}/all-rooms | ProjectController@allRooms | sanctum | All rooms in project |
| GET | /api/projects/{project}/shiftable-components | ProjectController@shiftableComponents | sanctum | All shiftable loads |
| POST | /api/projects/{project}/optimize-shiftable | ProjectController@optimizeShiftable | sanctum+admin/main | Run schedule optimizer |
| GET | /api/projects/{project}/defense-summary | ProjectController@defenseSummary | sanctum | Defense prep data |
| GET | /api/projects/{project}/backup | ProjectBackupController@backup | sanctum+admin/main | Export project JSON |
| POST | /api/projects/restore | ProjectBackupController@restore | sanctum | Import project JSON |
| GET | /api/buildings/{building}/backup | ProjectBackupController@backupBuilding | sanctum+admin/main | Export building JSON |
| POST | /api/projects/{project}/buildings/restore | ProjectBackupController@restoreBuilding | sanctum+admin/main | Import building JSON |
| GET | /api/floors/{floor}/backup | ProjectBackupController@backupFloor | sanctum+admin/main | Export floor JSON |
| POST | /api/buildings/{building}/floors/restore | ProjectBackupController@restoreFloor | sanctum+admin/main | Import floor JSON |
| GET | /api/rooms/{room}/backup | ProjectBackupController@backupRoom | sanctum+admin/main | Export room JSON |
| POST | /api/floors/{floor}/rooms/restore | ProjectBackupController@restoreRoom | sanctum+admin/main | Import room JSON |
| POST | /api/projects/{project}/save-backup | ProjectBackupController@saveProjectToServer | sanctum+admin/main | Save to server |
| POST | /api/buildings/{building}/save-backup | ProjectBackupController@saveBuildingToServer | sanctum+admin/main | Save building to server |
| POST | /api/floors/{floor}/save-backup | ProjectBackupController@saveFloorToServer | sanctum+admin/main | Save floor to server |
| POST | /api/rooms/{room}/save-backup | ProjectBackupController@saveRoomToServer | sanctum+admin/main | Save room to server |
| GET | /api/projects/{project}/server-backups | ServerBackupController@index | sanctum+admin/main | List server backups |
| GET | /api/server-backups/{backup}/data | ServerBackupController@show | sanctum+admin/main | Get backup data |
| DELETE | /api/server-backups/{backup} | ServerBackupController@destroy | sanctum+admin/main | Delete server backup |
| POST | /api/projects/{project}/buildings/{building}/duplicate | ProjectBackupController@duplicateBuilding | sanctum+admin/main | Duplicate building |
| POST | /api/buildings/{building}/floors/{floor}/duplicate | ProjectBackupController@duplicateFloor | sanctum+admin/main | Duplicate floor |
| POST | /api/floors/{floor}/rooms/{room}/duplicate | ProjectBackupController@duplicateRoom | sanctum+admin/main | Duplicate room |
| GET | /api/projects/{project}/members | ProjectMemberController@index | sanctum | List project members |
| POST | /api/projects/{project}/members | ProjectMemberController@store | sanctum+admin | Add member |
| PUT | /api/projects/{project}/members/{member} | ProjectMemberController@update | sanctum+admin | Update member role |
| DELETE | /api/projects/{project}/members/{member} | ProjectMemberController@destroy | sanctum+admin | Remove member |
| GET | /api/projects/{project}/buildings | BuildingController@index | sanctum | List buildings |
| POST | /api/projects/{project}/buildings | BuildingController@store | sanctum+admin/main | Create building |
| PUT | /api/projects/{project}/buildings/{building} | BuildingController@update | sanctum+admin/main | Update building |
| DELETE | /api/projects/{project}/buildings/{building} | BuildingController@destroy | sanctum+admin/main | Delete building |
| GET | /api/buildings/{building}/floors | FloorController@index | sanctum | List floors |
| POST | /api/buildings/{building}/floors | FloorController@store | sanctum+admin/main | Create floor |
| PUT | /api/buildings/{building}/floors/{floor} | FloorController@update | sanctum+admin/main | Update floor |
| DELETE | /api/buildings/{building}/floors/{floor} | FloorController@destroy | sanctum+admin/main | Delete floor |
| GET | /api/floors/{floor}/rooms | RoomController@index | sanctum | List rooms |
| POST | /api/floors/{floor}/rooms | RoomController@store | sanctum+admin/main | Create room |
| PUT | /api/floors/{floor}/rooms/{room} | RoomController@update | sanctum+admin/main | Update room |
| DELETE | /api/floors/{floor}/rooms/{room} | RoomController@destroy | sanctum+admin/main | Delete room |
| GET | /api/component-types | ComponentTypeController@index | sanctum | List component types |
| GET | /api/projects/{project}/phase-balance | PhaseBalanceController@project | sanctum | Project phase balance |
| GET | /api/buildings/{building}/phase-balance | PhaseBalanceController@building | sanctum | Building phase balance |
| GET | /api/floors/{floor}/phase-balance | PhaseBalanceController@floor | sanctum | Floor phase balance |
| POST | /api/rooms/{room}/assign-phase | PhaseBalanceController@assignRoom | sanctum+admin/main | Assign room phase |
| POST | /api/buildings/{building}/apply-optimal-phase | PhaseBalanceController@applyOptimalBuilding | sanctum+admin/main | Apply greedy phase |
| GET | /api/projects/{project}/load-profile | LoadProfileController@project | sanctum | 24h load profile |
| GET | /api/projects/{project}/schedule | ScheduleController@project | sanctum | Full schedule + dispatch |
| GET | /api/projects/{project}/cost-signal | CostSignalController@show | sanctum | Cost signal array |
| GET | /api/projects/{project}/financial-analysis | FinancialController@show | sanctum | Financial analysis |
| GET | /api/projects/{project}/total-power | TotalPowerController@project | sanctum | Project power totals |
| GET | /api/buildings/{building}/total-power | TotalPowerController@building | sanctum | Building power totals |
| GET | /api/floors/{floor}/total-power | TotalPowerController@floor | sanctum | Floor power totals |
| GET | /api/rooms/{room}/total-power | TotalPowerController@room | sanctum | Room power totals |
| GET | /api/projects/{project}/components | ProjectComponentController@index | sanctum | List project components |
| POST | /api/projects/{project}/components | ProjectComponentController@store | sanctum+admin/main | Add project component |
| PUT | /api/projects/{project}/components/{component} | ProjectComponentController@update | sanctum+admin/main | Update component |
| DELETE | /api/projects/{project}/components/{component} | ProjectComponentController@destroy | sanctum+admin/main | Delete component |
| GET | /api/buildings/{building}/components | BuildingComponentController@index | sanctum | List building components |
| POST | /api/buildings/{building}/components | BuildingComponentController@store | sanctum+admin/main | Add building component |
| PUT | /api/buildings/{building}/components/{component} | BuildingComponentController@update | sanctum+admin/main | Update component |
| DELETE | /api/buildings/{building}/components/{component} | BuildingComponentController@destroy | sanctum+admin/main | Delete component |
| GET | /api/floors/{floor}/components | FloorComponentController@index | sanctum | List floor components |
| POST | /api/floors/{floor}/components | FloorComponentController@store | sanctum+admin/main | Add floor component |
| PUT | /api/floors/{floor}/components/{component} | FloorComponentController@update | sanctum+admin/main | Update component |
| DELETE | /api/floors/{floor}/components/{component} | FloorComponentController@destroy | sanctum+admin/main | Delete component |
| GET | /api/rooms/{room}/components | RoomComponentController@index | sanctum | List room components |
| POST | /api/rooms/{room}/components | RoomComponentController@store | sanctum+admin/main | Add room component |
| PUT | /api/rooms/{room}/components/{component} | RoomComponentController@update | sanctum+admin/main | Update component |
| DELETE | /api/rooms/{room}/components/{component} | RoomComponentController@destroy | sanctum+admin/main | Delete component |
| GET | /api/nav/{projectName} | NavigationController@project | sanctum | Project breadcrumb |
| GET | /api/nav/{projectName}/{buildingName} | NavigationController@building | sanctum | Building breadcrumb |
| GET | /api/nav/{projectName}/{buildingName}/{floorName} | NavigationController@floor | sanctum | Floor breadcrumb |
| GET | /api/nav/{projectName}/{buildingName}/{floorName}/{roomName} | NavigationController@room | sanctum | Room breadcrumb |
| GET | /api/{entity}/{id}/utility-lines | UtilityLineController@index | sanctum | List utility lines |
| POST | /api/{entity}/{id}/utility-lines | UtilityLineController@store | sanctum | Create utility line |
| PUT | /api/utility-lines/{line} | UtilityLineController@update | sanctum | Update utility line |
| DELETE | /api/utility-lines/{line} | UtilityLineController@destroy | sanctum | Delete utility line |
| GET | /api/{entity}/{id}/generator-lines | GeneratorLineController@index | sanctum | List generator lines |
| POST | /api/{entity}/{id}/generator-lines | GeneratorLineController@store | sanctum | Create generator line |
| PUT | /api/generator-lines/{line} | GeneratorLineController@update | sanctum | Update generator line |
| DELETE | /api/generator-lines/{line} | GeneratorLineController@destroy | sanctum | Delete generator line |
| GET | /api/{entity}/{id}/sockets | SocketController@index | sanctum | List sockets |
| POST | /api/{entity}/{id}/sockets | SocketController@store | sanctum | Create socket group |
| PUT | /api/sockets/{socket} | SocketController@update | sanctum | Update socket group |
| DELETE | /api/sockets/{socket} | SocketController@destroy | sanctum | Delete socket group |
| GET | /api/projects/{project}/solar-systems | SolarSystemController@index | sanctum | List solar systems |
| POST | /api/projects/{project}/solar-systems | SolarSystemController@store | sanctum+admin/main | Create solar system |
| PUT | /api/solar-systems/{solarSystem} | SolarSystemController@update | sanctum+admin/main | Update solar system |
| DELETE | /api/solar-systems/{solarSystem} | SolarSystemController@destroy | sanctum+admin/main | Delete solar system |
| GET | /api/projects/{project}/batteries | BatteryController@index | sanctum | List batteries |
| POST | /api/projects/{project}/batteries | BatteryController@store | sanctum+admin/main | Create battery bank |
| GET | /api/projects/{project}/battery-runtime | BatteryController@projectRuntime | sanctum | Battery runtime summary |
| GET | /api/batteries/{battery} | BatteryController@show | sanctum | Get battery detail |
| PUT | /api/batteries/{battery} | BatteryController@update | sanctum+admin/main | Update battery |
| DELETE | /api/batteries/{battery} | BatteryController@destroy | sanctum+admin/main | Delete battery |
| POST | /api/batteries/{battery}/reset-soc | BatteryController@resetSoc | sanctum+admin/main | Reset SOC to full |
| POST | /api/batteries/{battery}/runtime-at-load | BatteryController@runtimeAtLoad | sanctum | Runtime at given kW |
| GET | /api/battery-chemistry-defaults | BatteryController@chemistryDefaults | public | Chemistry presets |
| GET | /api/validation/case-study | ValidationController@show | sanctum | Run validation test |
| POST | /api/admin/login | AdminController@login | public | Admin login |
| GET | /api/admin/users | AdminController@users | sanctum+admin | List all users |
| PUT | /api/admin/users/{user} | AdminController@updateUser | sanctum+admin | Update user |
| DELETE | /api/admin/users/{user} | AdminController@deleteUser | sanctum+admin | Delete user |
| GET | /auth/google/redirect | GoogleController@redirect | public | Start Google OAuth |
| GET | /auth/google/callback | GoogleController@callback | public | OAuth callback |

### Table 2: All Electrical Formulas

| Name | Expression | Units | Standard | File |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apparent Power | S = sqrt(P² + Q²) | VA | IEC 60364-8-1 | TotalPowerController |  |  |  |  |
| Active Power | P = S × PF | W | IEC 60364-8-1 | all controllers |  |  |  |  |
| Reactive Power | Q = P × tan(arccos(PF)) | VAR | IEC 60364-8-1 | ValidationReferenceService |  |  |  |  |
| System PF | PF = P / S | — | PENRA | TotalPowerController |  |  |  |  |
| Diversity Chain | DF = DF_room × DF_r2f × DF_f2b × DF_project | — | IEC 60364-8-1 | DiversityFactorService |  |  |  |  |
| Socket Tier 1 | D = n × 200 × 1.00 (n≤10) | VA | BS 7671 | SocketDemandService |  |  |  |  |
| Socket Tier 2 | D += (n-10) × 200 × 0.75 (10<n≤20) | VA | BS 7671 | SocketDemandService |  |  |  |  |
| Socket Tier 3 | D += (n-20) × 200 × 0.40 (n>20) | VA | BS 7671 | SocketDemandService |  |  |  |  |
| Capacitor Bank | Q_cap = Q_sys - P×tan(arccos(0.95)) | VAR | IEC 60831 | ValidationController |  |  |  |  |
| Capacitor Value | C = (Q/3)/(2πfV²) × 1e6 | μF | IEC 60831 | ValidationController |  |  |  |  |
| Inrush 125% | I_design = I_rated × 1.25 | A | NEC Art.430 | TotalPowerController |  |  |  |  |
| Phase Imbalance | imb% = (I_max-I_min)/I_avg × 100 | % | IEC 60034-26 | PhaseBalanceController |  |  |  |  |
| Phase Phasor | I = VA/230 × e^j(θ_V-arccos(PF)) | A | IEC 60909 | PhaseBalanceController |  |  |  |  |
| Neutral Current |  | I_N | = | I_A+I_B+I_C | (complex sum) | A | Kirchhoff | PhaseBalanceController |
| Solar Capacity Estimate | P = A × 0.17 × 1000 × 0.75 | W | IEC 61853 | SolarIrradianceService |  |  |  |  |
| PSH Sinusoidal Peak | P_peak = P_cap × PSH × PR × π / (2×D) | W | IEC 61724 | SolarIrradianceService |  |  |  |  |
| Solar from GHI | P = (GHI/1000) × P_cap × PR | W | IEC 61724 | SolarIrradianceService |  |  |  |  |
| Solar Declination | δ = 23.45 × sin(360/365 × (DOY-81)) | degrees | Spencer 1971 | SolarIrradianceService |  |  |  |  |
| Sunrise/Sunset | cos(H) = -tan(lat)×tan(δ); T = 12±H/15 | hours | Spencer 1971 | SolarIrradianceService |  |  |  |  |
| PSH Interpolation | PSH = PSH_L + frac×(PSH_U-PSH_L) | kWh/m²/day | NASA POWER | SolarIrradianceService |  |  |  |  |
| SOC Update (charge) | SOC += P/C × sqrt(RTE) | fraction | IEC 62619 | SourceDispatchService |  |  |  |  |
| SOC Update (discharge) | SOC -= P/C / sqrt(RTE) | fraction | IEC 62619 | SourceDispatchService |  |  |  |  |
| Gen Flat-Rate Cost | C = (fuel_price × F_rated) / P_rated | $/kWh | ISO 8528 | GeneratorLine |  |  |  |  |
| Gen Affine F(P) | F(P) = F_0 + (F_rated-F_0)×P/P_rated | L/h | ISO 8528-10 | GeneratorLine |  |  |  |  |
| Gen Marginal Cost | MC = fuel_price×(F_rated-F_0)/P_rated | $/kWh | ISO 8528 | GeneratorLine |  |  |  |  |
| Solar Discount | cost_h = max(0, base × (1-0.9×solar_frac)) | $/kWh | — | ProjectController |  |  |  |  |
| Annual Energy | E_annual = E_daily × 365 | kWh | — | FinancialAnalysisService |  |  |  |  |
| Weighted Tariff | tariff_w = (off_h×T + peak_h×T_peak)/24 | $/kWh | — | FinancialAnalysisService |  |  |  |  |
| Annual Savings | savings = cost_without - cost_with_solar | $ | — | FinancialAnalysisService |  |  |  |  |
| Simple Payback | T = investment / savings | years | — | FinancialAnalysisService |  |  |  |  |
| LCOE | LCOE = (C_install + C_maint×N) / (E_solar×N) | $/kWh | NREL/IEA | FinancialAnalysisService |  |  |  |  |
| Panel Degradation | factor = (1-0.005)^year | — | IEC 61215 | FinancialAnalysisService |  |  |  |  |
| Battery Replacement | year = ceil(cycle_life/365 - age_years) | year | — | FinancialAnalysisService |  |  |  |  |
| Cumulative 25yr | cum(y) = -invest + sum(savings×0.995^i - repl_i) | $ | — | FinancialAnalysisService |  |  |  |  |
| Self-Consumption | SC% = (solar_used+batt_chrg)/solar_gen × 100 | % | — | SourceDispatchService |  |  |  |  |
| Backup Hours (max) | T = C_usable / P_max_discharge | h | — | Battery model |  |  |  |  |
| Backup Hours (avg) | T = C_usable / (P_max/2) | h | — | Battery model |  |  |  |  |

### Table 3: Diversity Factors by Building Type

| Building Type | Room→Floor | Floor→Building | × Project DF (0.70) = Min Chain DF |
| --- | --- | --- | --- |
| residential_house | 0.60 | 0.70 | 0.60×0.70×0.70 = 0.2940 |
| residential_apartment | 0.65 | 0.70 | 0.65×0.70×0.70 = 0.3185 |
| hotel | 0.65 | 0.70 | 0.65×0.70×0.70 = 0.3185 |
| office | 0.85 | 0.80 | 0.85×0.80×0.70 = 0.4760 |
| educational_school | 0.80 | 0.80 | 0.80×0.80×0.70 = 0.4480 |
| educational_university | 0.85 | 0.80 | 0.85×0.80×0.70 = 0.4760 |
| retail | 0.85 | 0.85 | 0.85×0.85×0.70 = 0.5058 |
| hospital | 0.90 | 0.90 | 0.90×0.90×0.70 = 0.5670 |
| industrial | 0.85 | 0.85 | 0.85×0.85×0.70 = 0.5058 |
| mosque_worship | 0.80 | 0.75 | 0.80×0.75×0.70 = 0.4200 |
| sports | 0.80 | 0.80 | 0.80×0.80×0.70 = 0.4480 |
| (default/unclassified) | 0.90 | 0.80 | 0.90×0.80×0.70 = 0.5040 |

Note: The minimum chain DF above assumes room coincidence = 1.0. Actual component DF = room_type_coincidence × chain_DF. Critical components always use DF = 1.0 regardless.

### Table 4: Room Coincidence Factors

| Room Type | Coincidence Factor | Standard |
| --- | --- | --- |
| server_room | 1.00 | IEC/CIBSE |
| operating_theater | 1.00 | IEC/CIBSE |
| laboratory | 0.90 | CIBSE |
| classroom | 0.85 | CIBSE |
| lecture_hall | 0.85 | CIBSE |
| workshop | 0.85 | CIBSE |
| retail_floor | 0.85 | CIBSE |
| office_open | 0.80 | CIBSE |
| gym_sports | 0.80 | CIBSE |
| kitchen_commercial | 0.75 | CIBSE |
| office_private | 0.75 | CIBSE |
| prayer_hall | 0.75 | CIBSE |
| reception_lobby | 0.70 | CIBSE |
| meeting_room | 0.70 | CIBSE |
| corridor | 0.60 | CIBSE |
| living_room | 0.60 | CIBSE |
| kitchen_residential | 0.55 | CIBSE |
| hotel_room | 0.50 | CIBSE |
| bedroom | 0.45 | CIBSE |
| warehouse_storage | 0.30 | CIBSE |
| bathroom | 0.25 | CIBSE |
| (default/unclassified) | 0.80 | IEC 60364-8-1 |

### Table 5: Battery Chemistry Presets

| Chemistry | DoD | RTE | C-charge | C-discharge | Cycles | Calendar Life | Degradation/yr |
| --- | --- | --- | --- | --- | --- | --- | --- |
| lead_acid_flooded | 0.50 | 0.80 | 0.10 C | 0.20 C | 500 | 5 yr | 5.0% |
| lead_acid_agm | 0.50 | 0.85 | 0.20 C | 0.30 C | 700 | 7 yr | 4.0% |
| lead_acid_gel | 0.50 | 0.85 | 0.15 C | 0.25 C | 800 | 8 yr | 3.5% |
| lithium_lfp | 0.90 | 0.95 | 0.50 C | 1.00 C | 4000 | 15 yr | 2.0% |
| lithium_nmc | 0.80 | 0.93 | 0.50 C | 1.00 C | 2500 | 10 yr | 2.5% |

### Table 6: System Constants

| Constant | Value | File | Source Standard |
| --- | --- | --- | --- |
| PANEL_DEGRADATION | 0.005/yr | FinancialAnalysisService | IEC 61215 |
| PROJECTION_YEARS | 25 | FinancialAnalysisService | Industry standard |
| DF_PROJECT | 0.70 | ScheduleController, FinancialAnalysisService | IEC 60364-8-1 |
| ROOF_COVERAGE_RATIO | 0.17 | SolarIrradianceService | IEC sizing practice |
| STC_IRRADIANCE_W | 1,000 W/m² | SolarIrradianceService | IEC 60904-3 |
| CAPACITY_ESTIMATE_PR | 0.75 | SolarIrradianceService | Conservative PR |
| PERFORMANCE_RATIO | 0.80 | SolarIrradianceService | IEC 61724 |
| GEN_OPTIMAL_MAX_LOAD | 0.85 (85%) | SourceDispatchService | ISO 8528 |
| INV_EFF | 0.95 | SourceDispatchService | Industry standard |
| BATTERY_DEGRADATION_COST | 0.01 $/kWh | CostSignalService | Economic proxy |
| UNMET_COST | 999.0 $/kWh | CostSignalService | Sentinel value |
| VOLT | 230 V | PhaseBalanceController | IEC 60038 |
| WARN_PCT | 10% | PhaseBalanceController | IEC 60034-26 |
| CRIT_PCT | 20% | PhaseBalanceController | IEC 60034-26 |
| SOCKET_ASSUMED_PF | 0.95 | PhaseBalanceController | BS 7671 |
| TARGET_PF | 0.95 | ValidationController | PENRA |
| CAP_STEP_KVAR | 0.5 kVAR | ValidationController | IEC 60831 |
| VOLTAGE_LL | 400 V | ValidationController | IEC 60038 |
| FREQUENCY | 50 Hz | ValidationController | IEC standard |
| OUTLET_VA | 200 VA | SocketDemandService | BS 7671 |
| NASA_TIMEOUT_SEC | 10 s | SolarIrradianceService | — |
| CACHE_DAYS | 30 days | SolarIrradianceService | — |
| TOLERANCE_PCT | 0.1% | ValidationController | — |
| no_load_fuel default | 0.30 × F_rated | GeneratorLine | ISO 8528-10 |

### Table 7: Cost Signal Ladder

| Condition | Cost $/kWh | Reason |
| --- | --- | --- |
| Solar generated > load (surplus) | 0.00 | Energy would be curtailed/wasted; cost = 0 |
| Battery discharge headroom > 0 | 0.01 | Stored solar energy; near-zero degradation cost only |
| Utility available, off-peak hours | tariff (e.g. 0.12) | Standard grid rate |
| Utility available, peak hours (peak_start ≤ h < peak_end) | peak_tariff (e.g. 0.22) | Time-of-use peak rate |
| Generator available (no utility) | marginal_cost = fuel_price×(F_rated-F_0)/P_rated | Only incremental fuel burned |
| No source available | 999.00 | Load shedding sentinel — never schedule here |

### Table 8: Frontend Routes

| Path | Page Component | Auth Required | Description |
| --- | --- | --- | --- |
| / | Redirect to /dashboard | Yes | Root redirect |
| /login | LoginPage | No (public) | Google OAuth initiation |
| /auth/callback | AuthCallbackPage | No (public) | OAuth token receipt |
| /admin/login | AdminLoginPage | No (public) | Admin email/password login |
| /admin/dashboard | AdminDashboardPage | No (admin token) | User management |
| /dashboard | DashboardPage | Yes | Project list + create |
| /validation | ValidationPage | Yes | Case-study validation results |
| /defense-prep | DefensePrepPage | Yes | Defense presentation prep |
| /projects/:projectId | ProjectPage | Yes (ProjectLayout) | Project overview + total power |
| /projects/:projectId/schedule | LoadSchedulePage | Yes (ProjectLayout) | 24h schedule + dispatch |
| /projects/:projectId/phase-balance | PhaseBalancePage | Yes (ProjectLayout) | Phase imbalance analysis |
| /projects/:projectId/financial | FinancialPage | Yes (ProjectLayout) | 25yr financial projection |
| /projects/:projectId/single-line | SingleLineDiagramPage | Yes (ProjectLayout) | SVG single-line diagram |
| /projects/:projectId/buildings/:buildingId | BuildingPage | Yes (ProjectLayout) | Building overview |
| /projects/:projectId/buildings/:buildingId/floors/:floorId | FloorPage | Yes (ProjectLayout) | Floor overview |
| /projects/:projectId/buildings/:buildingId/floors/:floorId/rooms/:roomId | NewRoomPage | Yes (ProjectLayout) | Room components |

### Table 9: PSH Table (Peak Sun Hours kWh/m²/day)

Data derived from NASA POWER ALLSKY_SFC_SW_DWN monthly averages, TMY 1991–2020. Southern hemisphere months are season-flipped by +6.

| Latitude | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0° | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 | 5.5 |
| 10° | 4.9 | 5.3 | 5.7 | 6.1 | 6.2 | 6.1 | 6.1 | 6.1 | 5.7 | 5.3 | 4.9 | 4.7 |
| 20° | 4.1 | 4.8 | 5.7 | 6.5 | 6.9 | 7.0 | 6.9 | 6.6 | 5.8 | 4.9 | 4.0 | 3.7 |
| 30° | 3.2 | 4.1 | 5.4 | 6.6 | 7.3 | 7.7 | 7.5 | 6.9 | 5.8 | 4.6 | 3.2 | 2.8 |
| 40° | 2.0 | 3.1 | 4.8 | 6.3 | 7.5 | 8.0 | 7.7 | 6.7 | 5.3 | 3.8 | 2.2 | 1.7 |
| 50° | 0.8 | 1.9 | 3.8 | 5.7 | 7.3 | 8.0 | 7.5 | 6.1 | 4.3 | 2.7 | 1.0 | 0.5 |
| 60° | 0.0 | 0.9 | 2.7 | 5.0 | 7.0 | 8.0 | 7.2 | 5.2 | 3.1 | 1.4 | 0.1 | 0.0 |

Intermediate latitudes are linearly interpolated between the bounding rows.

*End of POWER_PROFILE_COMPLETE_DOCUMENTATION.md*

*Source: E:\graduation project\power-profile*

*Generated: 2026-06-07*